README.DOC for Crystal Semiconductor CS4232/CS4236 Windows 95 Drivers

This Document contains the following:

1. Version Numbers

2. List of files

3. File Summary

4. Installing Crystal Audio Drivers

5. Uninstalling Crystal Audio Drivers

6. Dos Mode

7. Multiple Crystal Audio Cards

1. Version Numbers:

1.60 	CS4232C.EXE

1.60 	CSFM.DRV

1.53 	CS32MIX.EXE

1.60 	CS32BA11.DRV

1.60 	CWAUDIO.VXD

1.60 	CS4232LD.VXD

1.60  CWAUDIO.BIN

79   	CS4232 microcode

6     CS4232A microcode

46	CS4236 microcode

1.54	SETUP.EXE

1.60  CSMIDI.DRV

1.60 	CSMIDI.VXD

1.51	CSPCPLE.DLL

1.53	CS32MIX.EXE

1.000	WSTREAM.DLL

1.000	MSGLOOP.EXE

1.000 VWSTREAM.VXD

2. List of Files:

CWAUDIO.BIN          6,349  06-10-96  4:00p

CSPCPLE.DLL         73,216  04-18-96  1:51a

WSTREAM.DLL          9,712  04-17-96  1:00a

CS32BA11.DRV       140,464  06-25-96  4:00p

CSFM.DRV            21,664  06-25-96  4:00p

CSMIDI.DRV           9,296  04-15-96  1:51a

CS32MIX.EXE        234,538  06-26-96  1:53a

CS4232C.EXE         85,296  06-19-96  4:00p

MSGLOOP.EXE          4,112  04-17-96  1:00a

SETUP.EXE          256,000  06-14-96  1:54a

CS4232.INF          14,500  06-21-96  4:00p

CS4232.INI             774  10-02-95  4:00p

CS4232LD.VXD        11,844  06-25-96  4:00p

CSMIDI.VXD          28,844  05-02-96  1:51a

CWAUDIO.VXD         85,047  06-25-96  4:00p

VWSTREAM.VXD         5,175  04-18-96  1:00a

README.DOC

3. File Summary:

CS4232C.EXE   	CS4232/CS4236 Configuration Program.  A DOS device driver used to initialize and configure the card.  It will download microcode from CWAUDIO.BIN.  Using the /X option, it will download resource data from RESOURCE.BIN.

CS32MIX.EXE	DOS mixer used to set or adjust Sound Blaster mixer settings.  It will run as a GUI application or as a command line utility.

CWAUDIO.BIN  	Data file containing microcode for the CS4232, CS4232A, and CS4236.  It is used by CS4232LD.VXD and CS4232C.EXE.

CS4232.INI   	Default initialization parameters for CS4232C.EXE.  It is only used if PnP is overridden (/O) or if not using the Win 95 option on a non PnP system (no /W).

CS4232.INF  	Windows 95 installation file.  It will install the files, update the registry, install CS4232C.EXE /W in the CONFIG.SYS, and it will install CS4232C.EXE /O /R in the DOS mode CONFIG.SYS.

CSFM.DRV	Crystal FM MIDI driver.

CWAUDIO.VXD	CS4232/CS4236 Windows 95 virtual device driver.

CS32BA11.DRV	CS4232/CS4236 Windows 95 wave, aux, and mixer driver.

CS4232LD.VXD   	CS4232/CS4236 Windows 95 Device Loader VxD.  This driver will download microcode from the CWAUDIO.BIN file.

SETUP.EXE		CS4232/CS4236 Install utility.  This executable will

remove the Crystal Drivers, clean up all the Crystal

Audio registry entries and prompt for a reboot.

README.DOC		End user documentation

4. Installing Crystal Audio Drivers

1.Turn PC off and insert audio card into an ISA slot unless audio device is present in your motherboard.

2.Turn PC on and start up Win95.

3.Win95 is a PnP OS.  It automatically recognizes new PnP audio hardware and creates a database for the hardware.  The following shows the order in which Win95 detects the Crystal logical devices:

d.Win95 detects the Crystal PnP Audio System Codec.  Win95 asks for the manufacturer's Disk.  Please select the approriate drive.  If the Windows default driver is not grayed out, installing DirectSound games replaces Crystal drivers with Microsoft Windows Sound System drivers.

e.Win95 detects the Crystal PnP Audio System Joystick (Joystick in some platforms).

f.Win95 detects the Crystal PnP Audio System Control Registers (CS4232 Support Registers in some platforms.)

g.Win95 detects the Crystal PnP Audio System MPU-401 Compatible.

h.Win95 detects the Crystal PnP Audio System CD-ROM.  This device may not be found if the Plug and Play resources do not reference a CD.  If your audio device is not using the CD-ROM interface, this device will not be detected.

If any of the above (a thru d) is not detected, a previous Win95 sound card installation was done.  For a clean reinstallation, the user should see the Uninstalling Crystal Audio drivers section below.

5. Uninstalling Crystal Audio Drivers

1.From Win95, run setup.exe provided in the Crystal installation diskette. The Setup will do an automatic cleaning up of the Win95 Crystal Audio registry entries. Many problems with reinstallation occur because of existing registry entries from previous installations and because of the \WINDOWS\INF\OEMx.INF files.  After running setup, the various CS4232 devices from the Control Panel should disappear.  It is recommended that prior to running Setup, remove the device entries in the Sound, Video, and Game Controller of the System Device Manager, and Crystal entries (if any) in the Other detected devices section of the Device Manager.

2.Shutdown and turn off your PC.

3.Turn PC on, Start Win95, Win95 will now detect new hardware, create an information database, and proceed with the installation of Crystal device drivers.

6. Dos Mode

To enable real DOS mode with CS4232/36 support:

1.Select a DOS prompt icon.

2.Select Properties.

3.Go to the Program tab.

4.Select the Advanced... button.

5.Click MS-DOS Mode.

6.Click "Specify a new MS-DOS configuration."

7.The following line will be automatically placed into the new MS-DOS configuration (with right path):

DEVICE=C:\WIN95\CS4232C.EXE /O

7. Multiple Crystal Audio Cards

To enable a CS4232/36 audio card in a system that

contains multiple CS4232/36 audio cards:

1.  Add the following option to the CS4232C.EXE

entry in the CONFIG.SYS.

Device=C:\WINDOWS\CS4232C.EXE /W /Sxx

where xx is the OEM ID number for the CS4232

audio card to be enabled.

2.  The OEM ID number can be obtained from the

manufacturer.