This is version 1.22 of the RVG386 driver, dated 6-Apr-96.    It is meant

for use with CPL    100VG/AnyLAN Ethernet cards running on NOVELL servers

and will run on both NOVELL version 3 and NOVELL version 4 servers.

Beginning with version 1.22,  the driver will run the various 100VG cards

in accordance with the following rules --

1)	ISA cards will always be run in "I-O" mode.

2)	Standard PCI cards will always be run in "I-O" mode.

3)  "Bus-master" PCI cards  will run in "bus-master" mode by default.

If the IOMODE keyword is specified, "I-O" mode will be used.

Note that the old "shared-memory" mode  has been removed from the version

1.22 driver (it did not improve ISA/EISA performance and in fact DEGRADED

performance of PCI cards!).     Note also that, for EISA and "bus-master"

cards, the used of "bus-master" mode is highly recommended due to its far

greater performance  ("I-O" mode should only be used  for diagnostics and

testing with these cards).

Loading the Driver --

The RVG386 driver must be loaded from the Novell server console  before a

100VG card may be used, as per the following sequence:

-- For NetWare 3.X --

---------------

SERVER

LOAD A:[path]LSLENH		     (Automatically loads module

PATCHMAN.NLM).

LOAD A:[path]RVG386 [parameters]     (Automatically loads MSM.NLM

and  ETHERTSM.NLM  from the

A:[path] subdirectory. Note

that these modules  are for

use with NetWare 3.X only).

After a workstation is logged onto the server, copy the following

Novell NetWare 386 V3.X modules  from diskette to the SYS:\SYSTEM

directory on the server --

RVG386.LAN

LSLENH.NLM

PATCHMAN.NLM

MSM.NLM

ETHERTSM.NLM

NetWare checks the SYS:\SYSTEM directory  while loading a driver,

and it will determine  that the above NLM modules also need to be

loaded.   The files LSLENH.NLM,  PATCHMAN.NLM,  TOKENTSM.NLM  and

MSM.NLM are usually provided by Novell but are also provided with

the 100VG driver diskette for convenience.

NOTE:  If you plan to load Novell's MONITOR.NLM, make certain its

version is 1.72 or later.    Prior versions are incompatible with

these modules and will cause the server to CRASH!

-- For NetWare 4.X --

---------------

Copy the files RVG386.LAN and RVG386.LDI from the driver diskette

to the SYS:\SERVER.4X subdirectory on the server.    You may then

use Novell's INSTALL routine,  or you may enter  the following to

load the driver:

C:\SERVER.4X\SERVER

LOAD C:\SERVER.4X\RVG386 [parameters]	     (Automatically loads

modules MSM.NLM and

ETHERTSM.NLM).

NOTE:  The NLM files provided on the diskette are only compatible

with NetWare version 3 and MUST NOT be used with version 4!!!

For more information on loading LAN drivers, refer to the "NOVELL Netware

System Administration"  or the  "NOVELL NetWare Installation and Upgrade"

manuals.   The parameters available for the RVG386.LAN driver are:

PORT=number

Specifies the base I-O port address of a card.  If this parameter

is omitted,  the driver displays a list  of I-O addresses for all

100VG cards present and lets the operator pick a card by entering

the desired I-O address.   For PCI cards, the address is a

hexadecimal number from 0400 to FF00 in increments of 0100 (i.e.,

the lower two digits must be 00 for PCI).

IOMODE

Forces a "bus-master" PCI card  to use "I-O" mode, instead of its

normal "bus-master" mode.     For all other cards, this parameter

is ignored.      Use of this parameter is NOT RECOMMENDED, except

when doing diagnostic work, as "bus-master" mode gives far better

performance.

INT=number

Specifies an override for the interrupt level programmed into the

EEPROM chip of ISA cards.    Use of this parameter is NOT

RECOMMENDED, except when doing diagnostic work (the card's EEPROM

should be programmed by the user with a suitable interrupt level,

to avoid confusion and system conflicts).     For PCI cards, this

parameter will be ignored, since PCI cards must use the interrupt

level assigned to them by the PCI BIOS.

FRAME=name

Specifies the frame type that the driver will use.   Valid values

are Ethernet_802.2, Ethernet_802.3, Ethernet_II or Ethernet_SNAP.

If this parameter is omitted, Ethernet_802.2 frames will be used.

NAME=board

Specifies a unique name of up to 17 characters for a board's con-

figuration.

NODE=number

Specifies an override to the node address  programmed in a card's

EEPROM.    This must be a hexadecimal number from 000000000000 to

FEFFFFFFFFFF;  note that the low-order bit of the first byte must

be zero.     Use of this parameter is NOT RECOMMENDED, except for

diagnostics -- if a different "locally-administered" node address

is required, the user should program that address into the card's

EEPROM, to avoid confusion and system conflicts.    If this para-

meter is omitted, the EEPROM node address will be used.

***** SPECIAL NOTE *****

When the V1.22 driver is for use with a "bus-master" PCI

running in "bus-master" mode,  the driver will reserve 30 of the server's

packet-input buffers (ECBs) for each card's input requirements.   This is

especially needed for the PCI cards, whose controller chips have only 32K

of RAM buffers (12K of which is used by the driver for transmissions) and

thus need additional CPU memory buffers to avoid losing input packets.

System administrators  of Novell servers  running the V1.22 server driver

may wish to configure the server with additional packet-input buffers for

better throughput, especially when multiple 100VG "bus-master" cards will

be used in a single server.   Highly-loaded Novell servers should have at

least 60 packet-input buffers allocated for each 100VG "bus-master" card,

which allows the server to do input processing while yet more packets are

being received by the hardware.

History of Changes:

Version 1.14 -- Revised keyword and card-scanning routines.   Removed 15-

second I-O timeouts.    Reworked driver to Novell version

3.2 driver specifications.

Version 1.15 -- Corrected interrupt-entry and PCI-scan routines.    Added

cable-disconnect testing and messages.

Version 1.16 -- Added full multicast-packet input filtering.

Version 1.17 -- Changed send-complete logic for frames issued when driver

is inactive (i.e. broken cable, driver "down", etc.).

Version 1.18 -- Revised interrupt logic to fix high-speed I-O problems.

Version 1.19 -- Added support for "bus-master" PCI cards.

Version 1.20 -- Corrected 10-MB problems with "bus-master" PCI cards.

Version 1.21 -- Corrected PCI setup error with Novell V4.xx servers.

Version 1.22 -- Deleted "shared-memory" logic for all cards.    Corrected

re-init logic and "bus-master" PCI card latency timings.