This is version 1.07 of the RVGNW driver, dated 14-Jun-96.    It is to be

used by CPL    100VG/AnyLAN Ethernet cards running in OS/2 workstations.

Beginning with version 1.07, all 100VG/AnyLAN cards shall be run in "I-O"

mode -- "shared-memory" mode was confusing to many users and did NOT give

a performance increase.   The old SHARED parameter for PCI cards has been

deleted from the V1.07 driver and should not be used in NET.CFG files.

For ISA and EISA cards, the user may override the IRQ level programmed on

the card by specifying INT=n in the run-time NET.CFG file, where n is the

desired IRQ level.   For PCI cards, the IRQ assigned by the PCI BIOS will

always be used, and any INT= parameter will be ignored.

100VG/AnyLAN PCI cards may now share an IRQ level with other devices,  if

required.   Note that this should be done only on "loaded" systems, where

there is no other way of configuring the system,  as sharing an IRQ level

by multiple devices is inefficient and can cost some performance.

Otherwise, the V1.07 driver operates as before  and may be used without a

NET.CFG file on systems having only a single 100VG card.    In this case,

the driver searches the system for  (a) the first PCI card, (b) the first

EISA card, or (c) an ISA card at address 300,  and the driver initializes

and runs whatever card it finds first.   For multiple-card systems or for

systems where different parameters apply, the user must specify a NET.CFG

file to set up a particular card properly.

Driver Installation --

The RVGNW.SYS driver is designed for use  with Novell's Netware Requester

for OS/2.    To install the driver, complete the steps outlined by Novell

under "Installing the NetWare Requester" in the Netware Requester manual.

Then, perform the following steps:

1)  Add the following line to the CONFIG.SYS file:

DEVICE=C:\directory\RVGNW.SYS

2)  Edit the NET.CFG file and add the following section:

Link Driver RVGNW

Port XXX              (where XXX is the I/O address

of the adapter.  Note: For EISA

the port is XC38 where X is the

slot that the adapter is located

in - 1C38 for slot 1, 2C38 for

slot 2, etc.)

3)  Copy RVGNW.SYS from the distribution diskette into the desired

directory.  This is usually \NETWARE.

4)   Make sure that the adapter and cabling are installed correctly

and reboot the computer.

Driver Revision History --

V1.05:  Minor corrections to the re-initialization logic.

V1.06:  Added PCI card capability.

V1.07:  Deleted "shared-memory" mode, corrected PCI card-scan logic.