The following instructions are for loading KTI Networks' 100VG AnyLAN adapters

with SCO OpenServer (Release 5).

1. Set up KTI 100VG adapter

(1) Boot your machine with DOS (version 3.0 or higher).

(2) Run KVGSETUP to configure the adapter.

2. Start X Windows

(1) Boot your machine with SCO OpenServer (Release 5) and login as root.

(2) Start X Windows by typing "startx &". If your system can't run X

Windows, you can run SCOADMIN instead to load and configure the driver,

the procedure is similar.

3. Install the software package on the floppy

(1) Open "Software Manager" by double clicking its icon.

(2) Go to "Software" --> "Install New".

(3) It asks "What host do you want to install from ?", Select local machine.

(4) A new window pops up to let you select media device, please insert the

driver disk for 100VG, then select "Floppy Disk Drive 0 (or 1)"

depending on your system.

(5) It displays "Installing: KTI 100VG Adapter Driver (Version 2.0.0a).

Do you want a FULL or PARTIAL installation", select "FULL".

(6) After the software is installed, "KTI 100VG Adapter Driver" should be

listed with other installed software.

4. Install the device driver

(1) Open "System Administration" --> "Networks" -->"Network Configuration

Manager".

(2) Goto "Hardware" --> "Add new LAN adapter". SCO will detect and list

100VG EISA and PCI adapters automatically. If the adapter is listed

correctly, then highlight it and select "Continue", skip to step 5;

if the adapter you have installed in your machine is not listed

correctly, select "Configure hardware not listed above", then you will

see a list of all supported network adapters, select one from the list

below:

KTI 100VG  ISA Adapter

KTI 100VG  PCI Adapter

KTI 100VG EISA Adapter

depending on the adapter installed in the system.

(3) ISA adapters:

Select the I/O address based on the setting you configured using

KVGSETUP. If there is only one 100VG ISA adapter installed, you may

also select "auto".

EISA adapter:

The slot number is required. SCO shows the slot number when it detects

a 100VG EISA adapter.

PCI adapters:

Both PCI bus number and device number are required. SCO shows the bus

number and device number when it detects a 100VG PCI adapter.

5. Install protocols

After the driver is configured, SCO will let you add protocols to it

one by one. If you want to access Novell server from SCO, both "SCO

IPX/SPX" and "SCO Gateway for Netware" are required, also make sure

the network number for "SCO IPX/SPX" matches the related configuration

in your Novell server.

6. Change priority

If it's required, the adapter can be set to work in high priority mode.

To do this, highlight the adapter first, go to "Hardware" --> "Modify

hardware configuration" --> "Advanced options", select "high" instead

of "standard".

7.  Re-link the kernel

When you exit from "Network Configuration Manager", you will be asked

if you want to re-link the kernel now. Choose yes,  set the new kernel

to boot by default, and rebuild the kernel environment.

Note:

(1) Each time when the system reboots, it displays one line for each 100VG

adapter similar to this:

device      address         vector  dma     comment

...

%kvg        0xE800-0xe81F   5       -       I/O mapped 00:40:f6:f8:80:d7

...

If the comment is "NO KTI BOARD PRESENT", you need to check the

configuration again, make sure it matches the setting in KVGSETUP.

(2) If you have more than one 100VG adapter installed in the system and

you want to remove one, we suggest you remove all adapters first in

"Network Configuration Manager", then reinstall again.