This file covers the following topics:

A. Installing the driver for IBM LAN Server 4.0 and OS/2 Requester

B. Installing the driver for IBM LAN Server 4.0 DOS LAN Service

C. Installing the driver for IBM LAN Server 3.0

D. Installing the driver for IBM LAN Server 3.0 DOS Client

A. Installing the driver for IBM LAN Server 4.0 and OS/2 Requester

------------------------------------------------------------------

1  In IBM LAN server, select or run MPTS (Multi-Protocol Transport Services)

program.

2  Press <Configure> button.

3  From the "Configure" dialog box, select <LAN Adapters and Protocols> option.

4  Press <Configure> button.

5  When "LAPS" dialog box is displayed, remove the current protocols and

adapter from the Current Configuration list box.

6  Since the KTI 100VG LAN adapter is not in the adapter list, press <Other

adapters> button.

7  Insert the KTI 100VG LAN adapter driver diskette into floppy drive A:.

8  Specify the drive and path for the driver as below:

A:\IBMLS\OS2\DP16I   for DP16I/DP16I2 ISA adapter series

A:\IBMLS\OS2\DP32E   for DP32E/DP32E2 EISA adapter series

A:\IBMLS\OS2\DP32PCI for DP32PCI/DP32PCI2 PCI adapter series

9  "KTI 100VG ISA LAN Adapter", "KTI 100VG EISA LAN Adapter", or

"KTI 100VG PCI LAN Adapter" will be moved into the adapter list.

10 Highlight your 100VG LAN adapter. Then press <Add> button to add it into the

Current Configuration list.

11 Add the required protocols for the adapter added.

12 Press <OK> button.

13 Press <Close> button.

14 Press <Exit> button to complete MPTS program.

15 The installation of the KTI 100VG LAN NDIS 2.01 driver for IBM LAN Server is

complete.

B. Installing the driver for IBM LAN Server 4.0 DOS LAN Service

---------------------------------------------------------------

1  Copy KTI 100VG NDIS 2.01 driver and installation files to the hard disk.

Create the C:\NDIS2 directory on the hard disk for the KTI 100VG driver

and installation files. Insert the driver diskette into the floppy drive

and enter the following commands at the DOS prompt:

MD C:\NDIS2

COPY A:\IBMLS\DOS\*.* C:\NDIS2

2  Install IBM LAN Server 4.0 DOS LAN Services program.

a) Insert "IBM LAN Server DOS LAN Services Disk 1" diskette into the

floppy disk drive and enter the following commands at the DOS prompt:

A:

INSTALL

b) When the installation program asks where the OEMSETUP.INF file

resides, answer with the path name where the HP driver files were

copied in step 2.  In this example, it should be:

C:\NDIS2

c) Then, follow the instructions on the screen and refer to the IBM

LAN Server 4.0 DOS Services Installation Guide, when needed, to

complete the installation procedure.

d) Let the installation program automatically update the CONFIG.SYS

and AUTOEXEC.BAT files to add the necessary commands to invoke the

DOS LAN Services program every time the computer is booted.

3  Reboot the computer.

Reboot the computer to have the changes in the CONFIG.SYS and

AUTOEXEC.BAT take effect.

4  The IBM LAN Server 4.0 DOS LAN Services installation is complete.

C. Installing the driver for IBM LAN Server 3.0

-----------------------------------------------

1  Insert the KTI 100VG LAN driver diskette into floppy drive A:.

2  In IBM LAN server, select or run the LAPS (LAN Adapter and Protocol Support)

program.

3  From the IBM LAPS dialog box, choose the <INSTALL> button to install a

KTI 100VG LAN adapter for the IBM LAN Server environment.

4  Specify the drive and path, A:\IBMLS\OS2, to the KTI 100VG LAN adapter

driver diskette.

5  After installing/copying the appropriate NIF files from the driver diskette,

the IBM LAPS dialog box will appear again.

6  Select the <Configure>.

7  Choose the <Configure LAN transports> option and select <Continue>.

8  When Configure Workstation dialog box is displayed, remove the current

configuration from the Current Configuration list box.

9  Choose the one of the "KTI 100VG ISA LAN Adapter", "KTI 100VG EISA LAN Adapter", and

"KTI 100VG PCI LAN Adapter" selections from the list box. Then

choose <Add> to add the KTI 100VG LAN adapter to the Current Configuration

list box.

10 Highlight the appropriate protocols in the Protocol list box. Then

choose <Add> to add them to the Current Configuration list box.

11 Choose <OK>.

12 The installation of the NDIS 2.01 driver for IBM LAN Server is complete.

D. Installing the driver for IBM LAN Server DOS Client

------------------------------------------------------

1  Use DXMAID to install the protocol and adapter driver

for DOS Requester for the client.

2  Select WD driver as a temporary driver. Later, replace it with KTI 100VG

LAN Adapter.

3  DXMAID program created a sub-directory \lsp.

4  Copy KTI 100VG LAN adapter NDIS driver kvgnd2.dos from directory \IBMLS\DOS

in the Driver Diskette.

5  Edit the CONFIG.SYS to remove WD driver and insert pcntnd.dos driver line

as below:

........

device=\lsp\protman.dos /I:\lsp

NDIS driver -> device=\lsp\kvgnd2.dos

device=\lsp\dxma0mod.sys 001

device=\lsp\dxmj0mod.sys

lastdrive=Z

6  Edit the PROTOCOL.INI to remove WD driver section and insert [KTIVG]

driver section. All bindings that refer to WD driver should be updated

to binding to [KTIVG] driver.

*** PROTOCOL.INI ***

[PROTMAN_MOD]

DriverName = PROTMAN$

[DXMJ0MOD_MOD]

DriverName = NETBEUI$

Bind to KTIVG ->    Bindings = KTIVG

NCBS = 15

ADAPTRATE = 1000

WINDOWERROR = 0

PIGGYBACKACKS = 1

DATAGRAMPACKETS = 2

PACKETS = 50

PIPELINE = 20

MAXTRANSMITS = 6

MINTRANSMITS = 2

LANABASE = 0

CHAINX5C = 0

[KTIVG]

Drivername = kvgnd2$

;       IOADDRESS = 0x300

;       SLOT = 5

;       BOARD = 1

;       MODE = IO

;       MODE = MEM

;       RAMADDRESS = 0xD0000

7  Now, your computer has been configured to use KTI 100VG LAN Adapter for

IBM LAN Server 3.0.