Using High Priority Transmission

Overview

--------

The KTI 100VG ISA, EISA and PCI LAN adapters provide the ability to manually

configure high priority operation on those computers that require it.

Normal priority operation on a 100VG LAN provides an end node with high

performance. Only specific types of applications require high priority

operation. In order not to block the 100VG bandwidth by high priority traffic,

you should first consult your network administrator and obtain authorization

before setting your computer to use high priority bandwidth.

Refer to the following considerations in design and configuration of your

network:

. Only those end nodes (clients or servers) that require real-time, or

time-critical access, may be configured for high priority.

End nodes that transmit bursts of network traffic, such as file transfers,

or data base access, should be configured for normal priority.

. End nodes that are manually configured for high priority operation will

always request high priority access to the network regardless of the

application. Remember to configure the end node back to normal priority

operation when you finish the high-priority-demanding application.

. Limit the amount of bandwidth for high-priority applications.

Future developments in Network Operating systems and applications will allow

the software to automatically determine the priority level on a per packet

basis, configuring high priority for time-sensitive applications and normal

priority for regular network access applications.  However, for the time being

such software does not exist so you must configure your adapter manually.

Manually configuring your adapter for high priority transmission forces all

packet transmission as high priority.

The current drivers provided in this driver diskette transmit each packet

as normal priority when the adapter is configured for normal priority and

transmit all packets as high priority when the adapter is configured as high

priority. You can manually configure priority level using the keyword or

parameter defined for the drivers. But, before using the priority keyword

for the drivers, you must consult your LAN administrator to obtain the

authorization.

DOS ODI Drivers

---------------

Both KVGODI.COM and KVGODIBM.COM drivers provide a "PRIORITY" keyword to

specify the priority level for the packet transmssion. With no keyword

specified, normal priority  is used.

You may specify the keyword in NET.CFG for high priority transmission

for your time-critical applications:

Link Driver KVGODI

PRIORITY        HIGH

Link Driver KVGODIBM

PRIORITY        HIGH

In order to free the high priority bandwidth for the network, you must

manually configure you computer back to normal priority each time you

finish the time-critical application. Please unload your network driver and

modify the NET.CFG file to set the priority back to normal and load the driver

again. The NET.CFG file can be modified as below:

Link Driver KVGODI

#       PRIORITY        HIGH

Link Driver KVGODIBM

#       PRIORITY        HIGH

or

Link Driver KVGODI

PRIORITY        NORMAL

Link Driver KVGODIBM

PRIORITY        NORMAL

Please refer to \DOS\INSTALL.DOC for more information.

Netware Server Driver

---------------------

Both Netware server drivers (KVGNW.LAN and KVGNWBM.LAN) provide a parameter

option for the transmission priority. The parameter name is "priority". As long

as "priority=high" is specified in the driver command line, all packets are

transmitted as high priority.

Command line for high priority transmission:

:load KVGNW priority=high

or

:load KVGNWBM priority=high

If high priority transmission is no longer required, unload the driver and load

the driver again with no "priority" parameter specified. Or, specify

"priority=normal" in the driver command line. See the following examples:

:load KVGNW

or

:load KVGNW priority=normal

Please refer to \NW4X\INSTALL.DOC for more information.