INSTALLING 100VG LAN ADAPTER DRIVER

FOR WINDOWS-FOR-WORKGROUPS VER. 3.11

====================================

CONFIGURE THE 100VG NETWORK DRIVER FOR A NEW WINDOWS

----------------------------------------------------

To configure the 100VG network driver for a new Windows, please follow the

on-screen instruction for "Windows Setup" procedure until a "Network Setup"

Window pop-up.

1. Select "Networks..." button for installing the network adapter driver.

2. Select "Install Microsoft Windows Network:" and "Ok".

3. Select "Drivers" button.

4. Select "Add Adapter" button.

5. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

6. System prompts you to insert driver diskette and enter the path of

the driver directory.

7. Insert the driver diskette into drive A: and enter A:\WFW311.

8. The following three items appear in the list:

"DP16I 100VG ISA LAN Adapter"

"DP32E 100VG EISA LAN Adapter"

"DP32PCI 100VG PCI LAN Adapter"

Select the right one for your installation.

9. Select "OK" button.

10. "DPxxx 100VG xxx LAN Adapter [NDIS2]" bar appears. It indicates the

NDIS2 driver to be used and copied into your Windows system.

11. Select "Setup" button to review more information.

12. Ignore the system pre-defined "Hardware Settings" on the screen.

At this step, you may select the "Driver type". Either "Real Mode NDIS

Driver" or "ODI driver" can be used.

13. If you want to change any configuration other than the setting

stored in EEPROM, you may select "Advanced" button to change it.

Otherwise, go to step 20.

This "Advanced" option is available only when "Real Mode NDIS Driver"

is selected. If "ODI Driver" is selected, you must modify your NET.CFG

to change the configuration or use KVGSETUP program to change the EEPROM

settings.

14. Select "OK" button to complete "Setup" window.

15. Select "Close" to finish "Drivers" window.

16. Now, the network adapter and driver have been added into your Windows.

Continue to follow the instruction for the rest Windows installation.

ADD 100VG ADAPTER AND NETWORK DRIVER TO YOUR EXISTING WINDOWS

-------------------------------------------------------------

Procedure :

It is assumed that the Windows for Workgroups 3.11 has been setup in

your computer without any network driver installed.

You may use either NDIS2 driver or Netware ODI driver for your installation.

If ODI driver is used, you have to start Netware DOS client softwares before

you start Windows for Workgroup 3.11. Refer to \DOS\install.doc file in this

driver diskette for more information on how to setup the Netware DOS client

software using ODI driver.

The following procedure only describes how to setup the network driver

for 100VG adapter:

1. Turn your computer off.

2. Install the 100VG LAN adapter into your computer.

3. After the adapter installation, turn your computer on.

4. Make sure your computer is under DOS.

5. Start your Windows for Workgroups 3.11.

6. Select "Program Manager".

7. Select "Network" icon.

8. Select "Network Setup" icon.

9. Select "Drivers" button.

10. Select "Add Adapter" button.

11. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

12. System prompts you to insert driver diskette and enter the path of

the driver directory.

13. Insert the driver diskette into drive A: and enter A:\WFW311.

14. The following three items appear in the list.

"DP16I 100VG ISA LAN Adapter"

"DP32E 100VG EISA LAN Adapter"

"DP32PCI 100VG PCI LAN Adapter"

Select the right one for your installation.

15. Select "OK" button.

16. "DPxxx 100VG xxx LAN Adapter [NDIS2]" bar appears. It indicates the

NDIS2 driver to be used and copied into your Windows system.

17. Select "Setup" button to review more information.

18. Ignore the system pre-defined "Hardware Settings" on the screen.

At this step, you may select the "Driver type". Either "Real Mode NDIS

Driver" or "ODI driver" can be used.

19. If you want to change any configuration other than the setting

stored in EEPROM, you may select "Advanced" button to change it.

Otherwise, go to step 20.

This "Advanced" option is available only when "Real Mode NDIS Driver"

is selected. If "ODI Driver" is selected, you must modify your NET.CFG

to change the configuration or use KVGSETUP program to change the EEPROM

settings.

20. Select "OK" button to complete "Setup" window.

21. Select "Close" to finish "Drivers" window.

22. Now, the network adapter and driver have been added into your Windows.

23. Follow the on-screen instruction to complete "Windows Setup".