December 1991

LCS-8834P Ethernet Pocket LAN Adapter

NetBIOS Emulator Installation

==================================

The LONGSHINE NetBIOS Emulator is a standardized network interface

program that allows this Adapter to run NetBIOS-based network

operating systems like the IBM PC LAN Program and others.  No

software modification or additional installation is necessary.

All operations as well as installation procedures for the network

operating system will remain unaltered.

The following files will be used in this installation:

Files           Contents

-----------     ----------------------

LCS8834P.EXE    LCS-8834P Driver program

LCSBIOS.EXE     NetBIOS Emulator Program

To install the NetBIOS Emulator, do as follows:

1. Load in the driver program LCS8834P.EXE.

>LCS8834P

2. Load in the NetBIOS Emulator program LCSBIOS.EXE.  This program

is memory-resident and will stay in your computer's memory

until its power is turned off or until you re-boot.

>LCSBIOS [/Sm] [/Ck]

Typing the parameter after LCSBIOS is optional.  The parameter is used

as follows:

/Sm   specifies the maximum number of sessions allowed on your

computer, where m can be a value from 1 to 254.  If this

parameter is not specified, the default number of sessions

will be set to 32.

/Ck   specifies the maximum number of NetBIOS command available for

your computer where k can be a value from 1 to 255.  If this

parameter is not specified, the default number of NetBIOS

commands will be set to 32.

Your computer now is ready to run a NetBIOS-based network

operating system.