# Chapter 7 - TAM API Reference

This chapter is a complete reference to the Mwave TAM Application Program Interface (API).

TAM functionality is provided by two separate, but related, drivers:  TAM Phone Line and TAM Message.

·	The Phone Line driver is used for all operations involving the phone line.  This includes playing a recorded message to the phone line, recording a message from the phone line, initiating calls, answering calls and speakerphone operation.

·	The Message driver includes supports all TAM operations that do not involve the phone line.  This includes local (i.e. phone line not used) recording and playing of messages.

This chapter is divided into two parts.  The first part describes the event messages issued by the two drivers and the second part describes the API messages and flags for the two drivers.  For the most part, the event messages and API messages/flags are identical for the two drivers.  Where the description is specific to a particular driver, the description is marked as follows:

MSG	Applicable to Message driver only

PL	Applicable to Phone Line driver only

Descriptions containing neither mark are applicable to both drivers.

## MCI Telephone Event Handler

Communication of real-time status information from the TAM driver to the application is performed through an application event handler. The handler should be able to service messages posted by the TAM driver through the MCI device, which contain real-time status information about the device.

MM_MCIEVENT, is not a standard MCI message under Microsoft Windows, thus a Microsoft Windows application must call the RegisterWindowMessage function with the string "MM_MCIEVENT", to obtain the numeric value of the notification message.

MM_MCIEVENT - Windows

In addition to the message itself, wParam and lParam are used to pass information to the application.

WPARAM   wParam

Contains a device specific event message wEvent.

LPMCI_EVENT_PARMS    lParam

Specifies a far pointer to the following MCI_EVENT_PARMS structure:

typedef struct {

DWORD	dwDataParam1;

DWORD	dwEventData;

} MCI_EVENT_PARMS;

The data parameters are defined as follows:

DWORD   dwDataParam1

The low-order word specifies the device specific event message wEvent (same as wParam). The high-order word specifies the device ID of the device initiating the message.

DWORD   dwEventData

Contains a data parameter, which is dependent on the message type. The actual parameters passed are listed in Table 1 below, and detailed in the event message descriptions.

MM_MCIEVENT - OS/2

In addition to the message itself, wParam and lParam are used to pass information to 			the application.

DWORD    MsgParam1

Contains a device-specific event message and device ID.

WORD    wEvent

The low-order word of MsgParam1 specifies the device-specific event code 	(same as usEventCode or wParam)

WORD wDeviceID

The high-order word of MsgParam1 specifies the device ID of the device 	initiating the message.

LPMCI_EVENT_PARMS    MsgParam2

typedef struct {

DWORD	dwDataParam1;

DWORD	dwEventData;

} MCI_EVENT_PARMS;

Note: The low-order word of dwDataParam1 contains the event code 	(same as wEvent).  The high-order word is not defined.

### TAM Event Message Descriptions

This section describes the Event Messages generated by the TAM API. The following table lists the Event Messages (wEvent), a short description of the data parameters, and the  associated drivers.

| Event Message (wEvent) | Data Parameter (dwEventData) | Driver(s) |
| --- | --- | --- |
| PHONE_EVENT_ADVANCED_RING | undefined, use lParam | PL |
| PHONE_EVENT_CALL_PROGRESS | New call state | PL |
| PHONE_EVENT_CALL_TAM | undefined | PL |
| PHONE_EVENT_CALL_TERMINATED | Call termination status | PL |
| PHONE_EVENT_CALLER_ID | Caller ID Status | PL |
| PHONE_EVENT_DISTINCTIVE_RING | Ring Identifier | PL |
| PHONE_EVENT_HANDSET | Handset Status | PL, MSG |
| PHONE_EVENT_HANDSET_KEY | Keypress character | MSG |
| PHONE_EVENT_LINE | Telephone line status | PL |
| PHONE_EVENT_LINE_KEY | Keypress character | PL |
| PHONE_EVENT_RING | Telephone ring status | PL |

Table 7-1:  TAM Driver Event Messages1

For all messages posted to the event handler routine, the message value is MM_MCIEVENT. The value of wEvent and dwEventData vary according to the specific message posted. Below is a more detailed description of the event messages and their parameters.

Arguments	wEvent:	PHONE_EVENT_ADVANCED_RING	PL

dwEventData:	not used, actual ring count is in lParam

Description	If the application has requested ‘advanced format ring notifications’ by setting advanced ring notify to TRUE, PHONE_EVENT_ADVANCED_RING is sent to the application instead of PHONE_EVENT_RING.  In this case, lParam is not a pointer to a structure.  Instead, the low word of lParam contains the ring count, and the high word of lParam contains the device ID.

LOWORD(lParam) =  0  Telephone ring signal end (not ringing)

LOWORD(lParam) = ‘n’	 Telephone ring count (where ‘n’ is the ring number)

Arguments	wEvent:	PHONE_EVENT_CALL_PROGRESS	PL

dwEventData:	dwCallProgress

Description	This message is posted when there has been a change in the current call state (or status). The new state of the call is supplied in dwCallProgress, and can be any of the following:

·	CALL_PROGRESS_ANSWER_TONE (supported in version 3.0 and above of the TAM driver)

·	CALL_PROGRESS_BUSY (in current driver, returned for both Fast Busy and Slow Busy)

·	CALL_PROGRESS_DIAL_TONE

·	CALL_PROGRESS_FAST_BUSY (unsupported in current driver)

·	CALL_PROGRESS_QUIET

·	CALL_PROGRESS_REMOTE_RINGING (supported in version 3.0 and above of the TAM driver)

·	CALL_PROGRESS_SLOW_BUSY (unsupported in current driver)

·	CALL_PROGRESS_UNIDENTIFIED_TONE

Arguments	wEvent:	PHONE_EVENT_CALL_TAM	PL

dwEventData:	undefined

Description	This message is posted when a call has been answered by the device, and has been determined to have originated from a voice source. At this time, the application can play a greeting and begin voice mail operations.

Arguments	wEvent:	PHONE_EVENT_CALL_TERMINATED	PL

dwEventData:	dwTermination

Description	This message is posted when a call has been terminated either by the caller, by the owning application, or because of an error condition. The reason for call termination is given in dwTermination, which can be any of the following values:

·	TERMINATION_ERROR_RECV

·	TERMINATION_ERROR_XMIT

·	TERMINATION_NORMAL

·	TERMINATION_REQUESTED (returned when the Discriminator is handing call off to a different driver)

·	TERMINATION_UNEXPECTED (returned if the PC goes into power saving mode in the middle of a call)

Arguments	wEvent:	PHONE_EVENT_CALLER_ID	PL

dwEventData:	dwCompStatus

Description	This message is posted when a caller ID string has been decoded off a ringing line. It is posted only if a caller ID signal is present. dwCompStatus indicates the completion status.

·	MCI_VALID_CALLER_ID_RECEIVED

·	MCI_CALLER_ID_FRAME_ERROR

The application must issue an MCI_INFO message to retrieve the id (for MCI_VALID_CALLER_ID_RECEIVED) or the error code (for

MCI_CALLER_ID_FRAME_ERROR).

Arguments	wEvent:	PHONE_EVENT_DISTINCTIVE_RING	PL

dwEventData:	dwRingIdentifier

Description	This message is posted when a distinctive ring has been decoded off a ringing line. It is posted only if distinctive ring support is installed. dwRingIdentifier indicates which distinctive ring has been decoded.  The ring identifier is a number between 1 and 20.  This support is added with Ver 3.2.

Arguments	wEvent:	PHONE_EVENT_HANDSET

dwEventData:	dwHandsetStatus

Description	This message is posted when the status of the telephone handset changes, due to the user either picking up or replacing the telephone handset. This message can be monitored to play an automatic greeting when the handset is removed from the cradle. The value of dwHandsetStatus is as follows:

dwHandsetStatus = 0	Handset is on-hook

dwHandsetStatus = 1	Handset is off-hook (in use)

Arguments	wEvent:	PHONE_EVENT_HANDSET_KEY

dwEventData:	dwKeypress

Description	This message is posted when a key has been pressed on the handset device. An ASCII character representing the pressed key ('0' - '9', 'a' - 'd', '#', '*', '!')  is supplied in dwKeypress.

The current PL driver reports only '!'. The '!' (flash) is reported only if the application has set the min and/or max flash time.

Arguments	wEvent:	PHONE_EVENT_LINE	PL

dwEventData:	dwLineStatus

Description	This message is posted when the status of the telephone line changes, due to another application in the system making use of the telephone line. When an application takes the telephone line off hook, or is called to service an incoming call, it remains in possession of the telephone line for the duration of the call. Applications which require use of the telephone line and find it busy, can simply wait for this message to signal that the telephone line may be used. The value of dwLineStatus is as follows:

dwLineStatus = 0	Telephone line is free

dwLineStatus = 1	Telephone line is in use

Arguments	wEvent:	PHONE_EVENT_LINE_KEY	PL

dwEventData:	dwKeypress

Description	This message is posted when a key has been pressed on the incoming telephone line. An ASCII character representing the pressed key ('0' - '9', 'a' - 'd', '#' or '*'), is supplied in dwKeypress.

Arguments	wEvent:	PHONE_EVENT_RING	PL

dwEventData:	dwRingStatus

Description	This message is posted when a ring signal change is detected by the device. This message can be used by the application to count ring cycles, or determine ring length. The value of dwRingStatus is as follows:

dwRingStatus = 0	Telephone ring signal end (not ringing)

dwRingStatus = 1	Telephone ring signal start (ringing)

## TAM Driver API Messages and Flags

This section describes the MCI compliant TAM API messages and flags. The following table lists MCI command messages used in the TAM API, a short description of the message and the associated drivers.

| MCI Message | Description | Drivers |
| --- | --- | --- |
| MCI_CLOSE | Close the device driver | PL, MSG |
| MCI_CONVERT | Convert from/to device dependent file to/from device independent file. | MSG |
| MCI_DIAL | Dial the telephone | PL |
| MCI_GETDEVCAPS | Get the capabilities of the device | PL, MSG |
| MCI_INFO | Get device string identifier | PL, MSG |
| MCI_LOAD | Load a voice or wave file for playing | PL, MSG |
| MCI_OPEN | Open the device driver | PL, MSG |
| MCI_PAUSE | Pause the voice or wave stream play or record | PL, MSG |
| MCI_PLAY | Play a voice or wave file | PL, MSG |
| MCI_RECORD | Record a voice or wave file | PL, MSG |
| MCI_RESUME | Resume a paused voice or wave stream | PL, MSG |
| MCI_SAVE | Save a recorded voice or wave file | PL, MSG |
| MCI_SEEK | Change the current position of the media | PL, MSG |
| MCI_SET | Configure the device | PL, MSG |
| MCI_STATUS | Query device configuration | PL, MSG |
| MCI_STOP | Stop a voice or wave stream | PL, MSG |

Table 7-2:  TAM Driver API Messages

MCI_CLOSE

This command message closes the TAM driver.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application. The event handler window must be specified in the dwCallback field regardless of whether MCI_NOTIFY or MCI_WAIT is selected.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Note: Be sure to assign the handle of the window procedure responsible for processing MM_MCINOTIFY messages to dwCallback prior to calling MCI_CLOSE regardless of whether MCI_WAIT or MCI_NOTIFY is specified.  Failure to do so results in erratic behavior when using versions earlier than Ver 2.1 of the TAM device driver.

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_CONVERT	MSG

This command message is used to convert data files between an MCI device dependent format, and a standard device independent format. The call is used to convert to and from device dependent format (PCM Wave in the case of Microsoft Windows) and TAM compressed voice files.

MCI_CONVERT is intended to be run ‘off-line’ as it consumes a fair amount of MIPS, and conversion time is the same as the duration of the file being converted.

MCI_CONVERT is supported in TAM drivers version 3.1 and above.  Support may not be installed on a system even if the driver version is 3.1 or above.  MCI_CONVERT will return a non-zero return code if it is not supported.  An AP can issue MCI_GETDEVCAPS to determine if wave file support is installed.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.  MCI_NOTIFY should be specified unless MCI_CONVERT_INFO is specified.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_CONVERT_CREATE

Indicates that the destination file is a new file which should be created. This will overwrite any existing file.

MCI_CONVERT_DESTINATION_FILE

Indicates the lpstrDestFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the destination file name.

MCI_CONVERT_DESTINATION_FORMAT

Indicates the dwDestFormat field of the data structure identified by lParam2 contains the desired format of the destination file. These include:

·	MCI_CONVERT_FMT_WAVE_PCM (from source of type DEVTAM)

·	MCI_TAM_CONVERT_FMT_DEVTAM (from WAVE_PCM)

MCI_CONVERT_DESTINATION_FROM

Specifies that a media starting position is included in the dwDestFrom field of the data structure identified by lParam2. This specifies the starting point at which the converted data is written to an existing destination file. This option is not supported with the MCI_CONVERT_CREATE option. For TAM, the index is in units of milliseconds.

MCI_CONVERT_INFO

Indicates that no conversion operation is to take place, but rather, the dwLength field of the data structure identified by lParam2 should be set to the length of the media of the supplied source device dependent filename. For a device dependent TAM file, the value is returned in milliseconds. If a device dependent file is not specified, this call returns an error.

MCI_CONVERT_LENGTH

Indicates that the dwLength field if the structure identified by lParam2 contains a value specifying the length of the media to be converted. If this value is not supplied, the entire media is converted from the starting index. For TAM, the length is expressed in units of milliseconds.

MCI_CONVERT_OVERWRITE

Indicates that newly converted information should overwrite any existing data. If this flag is not specified, the new data is inserted into the file.

MCI_CONVERT_SOURCE_FILE

Indicates the lpstrSrcFilename field of the data structure identified by lParam2 contains a pointer to the source file name.

MCI_CONVERT_SOURCE_FROM

Specifies that a media starting position is included in the dwSrcFrom field of the data structure identified by lParam2. This specifies the starting point at which the data to be converted is read from the source file. For TAM, the index is in units of milliseconds.

LPMCI_CONVERT_PARMS   lParam2

Specifies a far pointer to the following MCI_CONVERT_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR		lpstrDestFilename;

DWORD		dwDestFormat;

DWORD		dwDestFrom;

DWORD		dwLength;

LPCSTR		lpstrSrcFilename;

DWORD		dwSrcFrom;

} MCI_CONVERT_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_DIAL	PL

This command message takes the phone off-hook, and dials the supplied number. If the telephone is owned by another application at the time of this call, the command will fail.

Parameters	DWORD    lParam1

The following flags apply to the telephone device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_DIAL_DIALMODE

Specifies that the dwDialMode field of the data structure identified by lParam2 contains a constant specifying the phone dialing mode. Two modes are defined:

·	MCI_DIAL_MODE_PULSE

·	MCI_DIAL_MODE_TONE

MCI_DIAL_FLASH

Indicates that the telephone should be flashed before dialing the supplied number (if any).

MCI_DIAL_MONITOR

Specifies that the audio speaker device should be enabled during the calling process.

MCI_DIAL_STRING

Specifies that the lpstrDialString field of the data structure identified by lParam2 contains a pointer to a null terminated dialing string. Numeric characters '0' to '9' correspond to phone digits.  The '*' and '#' characters, the alpha characters 'a' to 'd'  and the '-' are also supported ('-' is ignored).

The 'w' character in the string specifies that the device should wait for a second dial tone before proceeding, and a ',' character indicates a pause in the dialing sequence. The time-out limit for the wait command (default 30 seconds) and the delay time for the pause command (default 2 seconds) are configurable using MCI_SET.  The ‘@’ character in the string specifies wait for quiet.  The ‘p’ character in the string specifies switch to pulse dialing.  The ‘t’ character in the string specifies switch to tone dialing.  The ‘!’ character in the string specifies flash the line.  Note that the setting of flash time has no effect on the duration of flash that is specified with a ‘!’ in the dial string.  That setting only has effect on the flash that occurs as a result of the MCI_DIAL_FLASH flag.

The maximum size string that can be dialed is specified by MAX_DIAL_STRING.

MCI_DIAL_VERIFY

Specifies that the call is to be verified. The phone is verified to be off-hook, and that a dial tone is present before dialing.

LPMCI_DIAL_PARMS  lParam2

Specifies a far pointer to the following MCI_DIAL_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwDialMode;

LPCSTR	lpstrDialString;

} MCI_DIAL_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_GETDEVCAPS

This command is used to obtain static information about a device.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_GETDEVCAPS_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which device capability to obtain. The following constants are defined:

MCI_GETDEVCAPS_CAN_EJECT

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_CAN_PLAY

The dwReturn field is set to TRUE if the device supports playing voice files to the speaker, handset, or telephone line. Otherwise, it is set to FALSE.

MCI_GETDEVCAPS_CAN_RECORD

The dwReturn field is set to TRUE if the device supports voice recording from the microphone, handset, or telephone line. Otherwise, it is set to FALSE.

MCI_GETDEVCAPS_CAN_SAVE

The dwReturn field is set to TRUE if the device supports saving voice data recorded from the microphone, handset, or telephone line. Otherwise, it is set to FALSE.

MCI_GETDEVCAPS_COMPOUND_DEVICE

The dwReturn field is set to FALSE prior to Ver 3.1.  For Ver 3.1 and beyond, it is set TRUE.

MCI_GETDEVCAPS_DEVICE_TYPE

The dwReturn field is set to MCI_DEVTYPE_OTHER.

MCI_GETDEVCAPS_HAS_AUDIO

The dwReturn field is set to TRUE if the device supports play and record through an external audio device (speaker and microphone). Otherwise, it is set to FALSE.

MCI_GETDEVCAPS_HAS_VIDEO

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_USES_FILES

The dwReturn field is set to TRUE if the device supports voice recording or playing. Otherwise, it is set to FALSE.

MCI_TAM_GETDEVCAPS_SUPPORTS_CUSTOM_TAG

The dwReturn field is set to TRUE if the TAM operations support custom audio file formats. These formats are intended to save disk space over the conventional PCM wave file format.

MCI_TAM_GETDEVCAPS_SUPPORTS_PCM_TAG

The dwReturn field is set to non-zero if the TAM operations support the use of standard PCM wave files in its play and record operations:   Otherwise it is set FALSE.  See MCI_SET for MCI_TAM_SET_LOW_LEVEL_WAVE_IO for related information.

LPMCI_GETDEVCAPS_PARMS   lParam2

Specifies a far pointer to the following MCI_GETDEVCAPS_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwReturn;

DWORD		dwItem;

} MCI_GETDEVCAPS_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_INFO

This command message obtains string information from the device.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_INFO_PRODUCT

Obtains a description of the hardware associated with a device. The description identifies both the driver and the hardware used. The string is copied to the buffer pointer to by the lpstrReturn field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure, and if the buffer is of insufficient size to contain the string, the string is truncated to fit the buffer.  The string contains a version number (i.e., “Ver 3.0”).  Driver enhancements will be denoted in this document with the “Ver x.y”. that corresponds with the first release that the feature shows up in.  The version number will always increase in future releases, so a program can parse the string, looking for “Ver “, convert the characters that follow “Ver “ to a number, and do a numeric greater-than-or-equal compare to determine if the function is available in the release the application is running with.

Note:  Unless otherwise noted, all functions are available as of Ver 2.2

MCI_INFO_CALLER_ID

Obtains a caller ID string.  (See PHONE_EVENT_CALLER_ID).  The string is copied into the buffer pointed to by the lpstrReturn (Windows) or dwReturn (OS/2) field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure (maximum size = MCI_MAX_CALLER_SIZE).  If the buffer is of insufficient size to contain the string, the string is truncated to fit the buffer.

The caller ID data is in the format defined by Bellcore's technical reference bulletin TR-TSY-000031 and TR-NWT-001188.  Also note that a checksum is included at the end of the Caller ID data.

MCI_INFO_CALLER_ID_ERROR

Obtains the caller ID error code. (See PHONE_EVENT_CALLER_ID). The code is copied into the buffer pointed to by the lpstrReturn (Windows) or dwReturn (OS/2)field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure.  The error code is either MCI_FRAME_ERROR or MCI_CHECKSUM_ERROR.

MCI_INFO_CALLER_PARSED_CALLER_ID

Obtains an already-parsed Caller ID string.  (See PHONE_EVENT_CALLER_ID).  The information is copied into the structure pointed to by the lpstrReturn (Windows) or dwReturn (OS/2) field of the structure identified by lParam2. The structure is:

typedef struct

{

char szDateTime[DATE_TIME_LEN+1];

char szNumber[MCI_MAX_CALLER_ID_SIZE]; /* callers number */

char szName[MCI_MAX_CALLER_ID_SIZE];    /* callers name (may      	                                                                     be null) */

} CIDINFO;

If the call doesn’t not have a caller ID, szName will be ‘out of area caller’ if  one phone system doesn’t support delivering caller ID to another phone system, or ‘private caller’ if the caller blocked the sending of caller ID.

This function is implemented in “Ver 3.0” of the TAM driver.

LPMCI_INFO_PARMS   lParam2

Specifies a far pointer to the following MCI_INFO_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPSTR		lpstrReturn;

DWORD		dwRetSize;

} MCI_INFO_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_LOAD

This command message loads a file, and the data used as the current media. The current position is set to the start of the media.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_LOAD_FILE

Indicates the lpstrFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the file name.

To open a new file, you can either:

·	Specify the MCI_LOAD_FILE and a null pointer for the file name.

·	If running with API style set to MCI_FAXTAM_STYLE_MMPM, omit the MCI_LOAD_FILE flag.

In driver version 3.1, the ability to play and record wave files over the telephone is added.  If the file extension is ‘wav’ the file is assumed to be a wave file.  If a new file is loaded, the value set by MCI_SET MCI_TAM_SET_FORMATTAG is used to determine that the file is a wave file.  The default setting is TAM_WAVE_FORMAT_CUSTOM.  Wave file support is a separately installable option that may not be installed on a particular machine.  If it is not installed, the application will get a non-zero return code on MCI_LOAD, MCI_PLAY or MCI_RECORD.  The TAM application can issue MCI_GETDEVCAPS for MCI_TAM_GETDEVCAPS_SUPPORTS_PCM_TAG to determine if support is installed.  Recording to wave files is not recommended as it takes much more disk space than recording to the custom formatted files.

Use LOADFILENAME instead of lpstrFilename (Windows) or pszElementName (OS/2).  This label makes it easier to port applications between Windows and OS/2.

MCI_OPEN_ELEMENT

This flag is defined in mciftdd.h to be identical MCI_LOAD_FILE.

LPMCI_LOAD_PARMS   lParam2

Specifies a far pointer to the following MCI_LOAD_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR	lpstrFilename;

} MCI_LOAD_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_OPEN

This command message initializes the telephony driver and hardware.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.  In older versions of the driver, the event handler window must be specified in the dwCallback field regardless of whether MCI_NOTIFY or MCI_WAIT is selected.

MCI_OPEN_ALIAS

Specifies that an alias is included in the lpstrAlias field of the data structure identified by lParam2. This command is handled by MCI.

MCI_OPEN_ELEMENT

Specifies that a filename is included in the lpstrElementName field of the data structure identified by lParam2. The file is loaded as part of MCI_OPEN processing.  This function is new to driver version 3.1.

MCI_OPEN_SHAREABLE

Specifies that the device should be opened as shareable.

MCI_OPEN_SHAREABLE is not supported in current TAM drivers.

MCI_OPEN_TYPE

Specifies that a device type name or constant is included in the lpstrDeviceType field of the data structure identified by lParam2. This command is handled by MCI.  To open the telephone message driver, specify "Mwavetps" in the lpstrDeviceType.  To open the telephone line driver, specify "Mwavetpl".

MCI_OPEN_TYPE_ID (Not supported in OS/2; defined as zero)

Specifies that the low-order word of the lpstrDeviceType field of the associated data structure contains a standard MCI device type ID and the high-order word optionally contains the ordinal index for the device. This command is handled by MCI.

LPMCI_OPEN_PARMS   lParam2

Specifies a far pointer to the following MCI_OPEN_PARMS data structure:

typedef struct {

DWORD		dwCallback;

WORD		wDeviceID;

WORD		wReserved0;

LPCSTR		lpstrDeviceType;

LPCSTR		lpstrElementName;

LPCSTR		lpstrAlias;

} MCI_OPEN_PARMS;

Note:	With Microsoft Windows, be sure to assign the handle of the window procedure responsible for processing MM_MCINOTIFY messages to dwCallback prior to calling MCI_OPEN regardless of whether MCI_WAIT or MCI_NOTIFY is specified.  Failure to do so results in erratic behavior when using versions earlier than Ver 2.1 of the TAM device driver.

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

Remarks

Case is ignored in the device name, but there must not be any leading or trailing blanks.

Note that the device type is the pszDeviceType field of the MCI_OPEN_PARMS data structure, but it does not have a corresponding flag because it is required and does not have a command-string parameter.

For the Mwave Fax and TAM drivers, the device types are:

Mwavetpl

Mwavetps

Mwavefax

MCI_PAUSE

This command message pauses the current MCI_PLAY or MCI_RECORD operation.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_PLAY

This command message plays the current media on the connected device(s).

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

If an MCI_NOTIFY_ABORTED is posted with the notification, the call discriminator determined that the call was not a voice call.  The application can now wait for the next incoming call.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_FROM

Specifies that a media starting position is included in the dwFrom field of the data structure identified by lParam2. The units assigned to the position values are milliseconds (MCI_FORMAT_MILLISECONDS). If MCI_FROM is not specified, the current position in the media is used.

MCI_TO

Specifies that a media ending position is included in the dwTo field of the data structure identified by lParam2. The units assigned to the position values are milliseconds (MCI_FORMAT_MILLISECONDS). If MCI_TO is not specified, the device  plays to the end of the media.

LPMCI_PLAY_PARMS   lParam2

Specifies a far pointer to the following MCI_PLAY_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwFrom;

DWORD		dwTo;

} MCI_PLAY_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_RECORD

This command message records the connected device(s) to the current media.

The ability to record a conversation is added in version 3.1 of the TAM driver.  This occurs if MCI_RECORD is issued when the handset is connected to the phoneline, and the handset is up, or if speakerphone is in use when MCI_RECORD is issued.  If the conversation is being recorded, the remote party will hear periodic beeps to indicate that the conversation is being recorded.  If the user wishes to change connections (i.e., from handset to  microphone during conversation recording) it is necessary for the application to issue MCI_STOP before issuing MCI_SET MCI_TAM_SET_CONNECT.  After the MCI_SET is complete, the application should issue MCI_RECORD without specifying MCI_FROM to continue recording from the position where the initial recording stopped

With Ver 3.1, the ability to record PCM files is supported.  However, PCM files take up more disk space than the default TAM sub-band-coded files.  Also, when recording PCM files over the phone line, neither silence nor dialtones are automatically removed from the recorded file. With sub-band-coded files, recording over the phone is automatically terminated when the call is complete. With PCM files, the application must issue MCI_STOP to terminate the record.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

If MCI_NOTIFY_ABORTED is posted with the notification, the call discriminator determined that the call was not a voice call.  The application should not save the recorded file.  If MCI_NOTIFY_FAILURE is reported, it probably indicates that nothing but silence was recorded.  There is no reason to save the recorded file.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_RECORD_INSERT

Indicates that newly recorded information should be inserted or pasted into the existing media data.

MCI_FROM

Specifies that a media starting position is included in the dwFrom field of the data structure identified by lParam2. The units assigned to the position values are milliseconds (MCI_FORMAT_MILLISECONDS). If MCI_FROM is not specified, the current position in the media is used.

MCI_RECORD_OVERWRITE

Specifies that newly recorded data should overwrite existing data.

MCI_TO

Specifies that a media ending position is included in the dwTo field of the data structure identified by lParam2. The units assigned to the position values are mS (MCI_FORMAT_MILLISECONDS). If MCI_TO is not specified, the device records to the end of the media (a substantial amount of time in TAM).

MCI_TAM_BEEP

Specifies that a 500 Hz tone of 0.5 second duration should be played before recording begins.

MCI_TAM_TO_MESSAGE_END

Specifies that the device should record until it detects the end of the message, and then truncates prolonged silence or dial tone periods from the newly recorded media. The MCI_TAM_TO_MESSAGE_END flag should always be set when not using the MCI_TO option.

LPMCI_RECORD_PARMS   lParam2

Specifies a far pointer to the following MCI_RECORD_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwFrom;

DWORD		dwTo;

} MCI_RECORD_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_RESUME

This command message resumes the current MCI_PLAY or MCI_RECORD operation, after a MCI_PAUSE operation has been issued.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_SAVE

This command message saves the current media to a file, retaining its current format via the format tag.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_SAVE_FILE

Indicates the lpstrFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the file name where the current media data is saved.

LPMCI_SAVE_PARMS   lParam2

Specifies a far pointer to the following MCI_SAVE_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR	lpstrFilename;

} MCI_SAVE_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_SEEK

This MCI command message changes the current position of the media. Audio output is disabled during the seek. After the seek completes, the device stops.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_SEEK_TO_END

Specifies that the device should seek to the end of the current media.

MCI_SEEK_TO_START

Specifies that the device should seek to the start of the current media.

MCI_TO

Specifies a position is included in the dwTo field of the structure identified by lParam2, to which the device should seek using the current media. Seek distance is specified in units of mS (MCI_FORMAT_MILLISECONDS).

LPMCI_SEEK_PARMS   lParam2

Specifies a far pointer to the following MCI_SEEK_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwTo;

} MCI_SEEK_PARMS;

Return Value	       Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_SET

This command is used to set TAM device information. The item to set is specified by dwItem field of the MCI_TAM_SET_PARMS structure, pointed to by lParam2, and set data information is passed in dwSetData.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_SET_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which item to set. The following constants are defined:

MCI_TAM_SET_ADVANCED_RING_NOTIFY

The dwSetData field is set to the indicate the type of message that is sent to the application when the phone rings. When set to FALSE (the default), a PHONE_EVENT_RING is sent to the application. When the flag is set to TRUE, a PHONE_EVENT_ADVANCED_RING is sent to the application.  With advanced format ring events, lParam does not contain a pointer to dwSetData.  Instead, lParam contains the device ID and the actual ring count (not a pointer to it).  A ring count of zero indicates the end of a ring.

MCI_TAM_SET_AP_DISCRIMINATED	PL

This function, new to driver version 3.1, gives the TAM application the ability to influence the call-discrimination outcome.  For example, a TAM application that has a caller ID database can indicate the incoming call is for FAX, MODEM, VOICE, or don’t answer.  The duration of the setting is for the current call only.

The application should preferably issue this call before the current call is answered.

The dwSetData  field is set to indicate MCI_FAXTAM_PASS_FAX, MCI_FAXTAM_PASS_MODEM, MCI_FAXTAM_PASS_VOICE, or MCI_FAXTAM_DONT_ANSWER.  If this call is issued before the discriminator discriminates, the AP’s preference will take precedence over any other discrimination criteria, and the discriminator will not discriminate based on calling tones or information in the discriminator’s database.

MCI_TAM_SET_API_STYLE

Specifies that the dwSetData field contains the API style of the TAM device.  The possible values are:

·	MCI_FAXTAM_STYLE_MMPM

·	MCI_FAXTAM_STYLE_WINDOWS

The default style under OS/2 is MMPM.  The default style under Windows is WINDOWS.  The API style affects return codes for MCI_STATUS, MCI_GETDEVCAPS and MCI_INFO.  The style also affects return codes and return information for MM_MCINOTIFY.  See Microsoft Windows Multimedia Programmer's Reference and IBM's Programming Reference for Multimedia Presentation Manager Toolkit/2 for details of the MCI interface as specified for Windows and OS/2.

MCI_TAM_SET_AUDIO_MUTE

The dwSetData field is set to the desired mute status of the system microphone. When set to TRUE, the microphone (audio input of TAM_AUDIO) is disconnected from the telephone line, and any record operation in progress. When set to FALSE, the device operates normally.

MCI_TAM_SET_AUDIO_VOLUME

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_TAM_SET_AVGBYTESPERSEC

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the desired average bytes per second rate of any new RECORD operation.

With Ver 3.1, PCM files are supported.  Average bytes per second used with sub-band-coded files is treated as follows:

For most messages with normal speech, the data rate observed will be close to this average. If this value is greater than zero, it overrides with a finer granularity the current MCI_TAM_SET_QUALITY level.

The minimum non-zero value that can be input is 1000. The actual level being used can be found by calling MCI_STATUS. If dwSetData is set to zero, the value of MCI_TAM_SET_QUALITY is used. This request does not effect message playback.

With PCM files, average bytes per second must be set to be consistent with bits per second and bits per sample.  The formula for calculating bytes per second is:

average bytes per second = bits per second * (bits per sample/8)

The default for average bytes per second is 11025.  The default for bits per second is 11025.  The default for bits per sample is 8.  A list of valid combinations includes:

| Bytes/second | Bits/sample | Samples/second |
| --- | --- | --- |
| 11025 | 8 | 11025 |
| 22050 | 16 | 11025 |
| 22050 | 8 | 22050 |
| 44100 | 16 | 22050 |
| 44100 | 8 | 44100 |
| 88200 | 16 | 44100 |
| TABLE 7-3 |  |  |

NOTE:  TAM_WAVE_FORMAT_CUSTOM (sub band coded) files are equivalent to 15 bits/sample at 11025 samples/second.  On average, sub band coded files take less than 4000 bytes/second of disk space.

MCI_TAM_SET_BITSPERSAMPLE

Sets the desired bits per sample (either 8 or 16) used for playing, recording, and saving to the dwSetData field of the data structure identified by lParam2. This command is used in conjunction with MCI_TAMSET_SAMPLESPERSEC for PCM format wave files only.  Using 16 bits per sample sounds noticeably better than 8 bits per sample, but uses twice the disk space.

Note: PCM format files are only supported by version 3.1 or above, of the  TAM driver.  See MCI_TAM_SET_AVGBYTESPERSEC for more information.

MCI_TAM_SET_CALLER_ID	PL

Specifies that the dwSetData field of the data structure identified by lParam2 is set to FALSE to disable caller ID processing;  otherwise, it is set to TRUE. Caller ID processing uses Mwave DSP resources. Disabling caller ID permits more concurrency.  The default is TRUE on systems that have Mwave call discrimination installed.

MCI_TAM_SET_CALL_FILTER	PL

Specifies that the dwSetData field of the data structure identified by lParam2 is set to TRUE if the device is to receive voice calls; otherwise it is set to FALSE. If another application has this filter enabled, attempting to enable the filter  causes an error return.

MCI_TAM_SET_CONNECT

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the new target play or record device, and any inter-device connections. The device flags for the devices to connect are OR'ed together, and the result is placed in dwSetData. The TAM device flags are defined as follows:

·	MCI_TAM_AUDIO (speaker & microphone)	MSG

·	MCI_TAM_HANDSET	 MSG

·	MCI_TAM_PHONELINE	 PL

·	MCI_TAM_AUDIO_PHONELINE	PL

(speakerphone)

·	MCI_TAM_HANDSET_PHONELINE	PL

(standard phone operation)

·	MCI_TAM_SPEAKER_PHONELINE	PL

(answering machine w/ call screening)

Note:  The device MCI_TAM_PHONELINE is required for all operations involving an outside phone line.

Note: Setting speakerphone operation disables call discrimination based on calling tones.  DTMF key detection is also disabled.

MCI_TAM_SET_DIAL_FLASH_TIME	PL

The dwSetData field is set to the desired flash time (in milliseconds) of the telephone flash option in the MCI_DIAL command. The default value is 500 (one half second).

MCI TAM_SET_DIAL_PAUSE_TIME

The dwSetData field is set to the desired pause time (in milliseconds) that an embedded ',' character produces in the dial string. The default value is 2000 (2 seconds).

MCI_TAM_SET_DIAL_WAIT_TIME	PL

The dwSetData field is set to the desired time-out limit (in milliseconds) that an embedded 'w' character in the dial string allows, waiting for a second dial tone. The default value is 30000 (30 seconds).

MCI_TAM_SET_EVENT_HANDLER

Specifies that the dwSetData field of the data structure identified by lParam2 contains the handle of the application event handler. The MCI driver posts MM_MCIEVENT messages when an event occurs which changes the status of the driver. Setting this value to zero disables event posting. See the event handler section of the document for more details.

MCI_TAM_SET_FORMATTAG

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the compression/format of the media to be played or recorded. The following formats are allowed:

·	WAVE_FORMAT_PCM (Supported by version 3.1 and above of the TAM drivers)

·	TAM_WAVE_FORMAT_CUSTOM

MCI_TAM_SET_HANDSET_MUTE	MSG

The dwSetData field is set to the desired mute status of the telephone handset. When set to TRUE, the audio input of the handset is disconnected from the telephone line, and any record operation in progress. When set to FALSE, the device operates normally.

MCI_TAM_SET_HANDSET_VOLUME	MSG

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_TAM_SET_HOOK	PL

The dwSetData field is set to the desired hook status of the telephone line. It is set to TRUE to take the handset off-hook, and FALSE to place the handset on-hook. If another application owns the phone line, and the value is set to TRUE, this call will fail. When an application sets dwSetData to FALSE, it relinquishes ownership of the line.

MCI_TAM_SET_LOW_LEVEL_WAVE_IO

The dwSetData field is set to inform TAM driver that the application intends to use the low level wave audio API to play or record from the phone.  This function is available with Ver 3.2 of the TAM drivers.  Valid values include:

·	MCI_TAM_WAVE_IN_START

·	MCI_TAM_WAVE_IN_STOP

·	MCI_TAM_WAVE_OUT_START

·	MCI_TAM_WAVE_OUT_STOP

In general, a TAM application that uses wave files will not use this                                                          interface.  However specialized applications, such as voice recognition applications, cannot wait until an entire file has been recorded and saved.  Those applications will want to analyze the PCM data as it arrives.   To examine buffers as they are received, the application must use the audio driver directly.

Under Windows, the set of calls that the application should make are MCI_GETDEVCAPS for MCI_TAM_GETDEVCAPS_SUPPORTS_PCM_TAG.  This returns the device ID of the wave driver that can play to the telephone or handset.  The application uses this device ID on the low level audio calls (e.g., waveOutSetVolume).  Before opening the wave driver, the application should call MCI_SET to inform the TAM driver that it is about to open the wave driver to start input or output.  Likewise, after closing the wave driver, MCI_SET is issued to inform the TAM driver that the low level audio is done.

Under MMPM (OS/2), there are no low level audio API’s.  However, if the application wants to inspect PCM buffers it must use the audio driver directly, and do I/O using memory playlists.  To accomplish this, the application issues MCI_SET to inform the TAM driver that the wave driver is going to be used (as above).  It then issues MCI_OPEN for the wave audio device.  After that, it issues ‘connection <alias> query type wave stream alias conndev wait’.  The connection command is followed by ‘connector conndev enable type phone line wait’.  If using the MSG driver, use ‘phone set’ instead of ‘phone line’.  After the application is done using the wave device, MCI_SET is issued to inform the TAM driver that the wave driver is no longer in use.

The application is responsible for setting the speaker volume if it uses the low level audio API.

Most applications will not need this interface.  To play or record wave files, it is much simpler to issue ‘load’,  and ‘play’ or ‘record’ directly to the TAM driver.

MCI_TAM_SET_MAX_FLASH_TIME	PL

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant maximum number of milliseconds used for detecting when a handset flash  has been pressed.  A flash essentially is an off-hook followed by an on-hook.  The period of time between the two events determines if the telco detects  one flash hook or two separate events (on-hook and off-hook). Different telcos may use different values. This call allows the application to adjust to the different telcos.  The default is zero, meaning that flash will not be reported to the application. When flash is detected, it is reported in a PHONE_EVENT_HANDSET_KEY, with the key value set to ‘!’.  If the max flash time is set less than the min flash time, it is treated as an error.

MCI_TAM_SET_MICROPHONE_GAIN

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the gain of the microphone in dB.  Valid values are from 0 to 100 decimal.  The default is 50 dB.

MCI_TAM_SET_MIN_FLASH_TIME	PL

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant indicating the smallest  number of milliseconds used for detecting when a handset flash  has been pressed.  A flash essentially is an off-hook followed by an on-hook.  The period of time between the two events determines if the telco detects one flash hook or two separate events (on-hook and off-hook).  Different telcos may use different values. This call allows the application to adjust to the different telcos.  The default is zero. When flash is detected, it is reported in a PHONE_EVENT_HANDSET_KEY, with the key value set to ‘!’. If the min flash time is set greater than the max flash time, it is treated as an error.

MCI_TAM_SET_PASS_CALL	PL

The dwSetData field is set to a constant indicating the desired type of application that the current phone call will be  passed to.  The possible values are:

·	MCI_FAXTAM_PASS_FAX to pass to a fax application

·	MCI_FAXTAM_PASS_MODEM to pass to a modem                     application

If the specified application is not currently accepting incoming calls, the application retains ownership of the call, and should remember to hang up the phone.

This call can work only if the Mwave call discriminator is active.

MCI_TAM_SET_QUALITY

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the quality level of the phone recording and playback. Quality range is (0-7), where "0" is lowest quality, and "7" is highest quality.

MCI_TAM_SET_QUIET_DURATION	PL

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the continuous phone line quiet time in seconds before the application will get the first MM_MCIEVENT message specifying CALL_PROGRESS_QUIET.  The minimum non-zero value is 4.  Zero indicates that the application doesn't want the CALL_PROGRESS_QUIET interrupt returned.  The default is 10.  After the first CALL_PROGRESS_QUIET, the application will continue receiving this message every second until the call is terminated.

MCI_TAM_SET_QUIET_DURATION is not supported in current TAM drivers.  In the interim, the first MM_MCIEVENT message specifying CALL_PROGRESS_QUIET is returned after 4.5 seconds of continuous phone line quiet time.

MCI_TAM_SET_RING_COUNT	PL

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the ring count at which the device should answer the telephone. The driver will answer on the shortest ring count request of all active applications. Setting this value to 0 requests that the telephone not be answered.  The default ring count for TAM is 3.

Prior to driver version 3.1 if the discriminator is loaded, it will answer the telephone on the shortest ring count request of all registered applications. In version 3.1 and above, the discriminator uses the phoneline application’s ring count if there is a phoneline application active.

The maximum ring count that can be set is specified by MAX_RING_COUNT.

MCI_TAM_SET_SAMPLESPERSEC

Sets the samples per second used for playing, recording, and saving to the dwSetData field of the data structure identified by lParam2. This is used for PCM format only.

MCI_TAM_SET_SAMPLESPERSEC is supported in version 3.1 and above of the TAM drivers. See MCI_TAM_SET_AVGBYTESPERSEC for more information.

MCI_TAM_SET_SPEED	MSG

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the speed to play to the current media device. The speed index passed in dwSetData is the play speed factor (1/32 to 2) multiplied by 32. Examples include (but are not limited to) the following:

·	16 	-  1/2 x normal speed

·	24 	-  3/4 x normal speed

·	32 	-  Normal (recorded) speed

·	40 	-  1.25 x normal speed

·	48	-  1.5 x normal speed

·	56	-  1.75 x normal speed

·	63	-  2x normal speed

LPMCI_TAM_SET_PARMS   lParam2

Specifies a far pointer to the following MCI_TAM_SET_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwSetData;

DWORD		dwItem;

} MCI_TAM_SET_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_STATUS

This command is used to obtain information about the TAM device. Information is returned in the dwReturn field of the MCI_STATUS_PARMS structure, pointed to by lParam2.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_STATUS_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which status item to obtain. The following constants are defined:

MCI_STATUS_CALLER_ID	PL

The dwReturn field is set to the status of the caller ID processing.  The following status caller ID are defined:

·	MCI_CALLER_ID_ACTIVE (Mwave support is installed and     loaded)

·	MCI_CALLER_ID_NOT_SUPPORTED (Mwave support not installed)

·	MCI_CALLER_ID_DISABLED (by application issuing MCI_SET_CALLER_ID FALSE or because Mwave is processing a fax or modem call)

MCI_STATUS_LENGTH

The dwReturn field is set to the length of the current play/record media in milliseconds.

MCI_STATUS_MODE

The dwReturn field is set to the current mode of the device. The following modes are defined:

·	MCI_MODE_NOT_READY

·	MCI_MODE_PAUSE

·	MCI_MODE_PLAY

·	MCI_MODE_STOP

·	MCI_MODE_OPEN

·	MCI_MODE_RECORD

·	MCI_MODE_SEEK

MCI_STATUS_POSITION

The dwReturn field is set to the current position of the play/record media in milliseconds.

MCI_STATUS_READY

The dwReturn field is set to TRUE if the device is ready to receive a call; otherwise, it is set to FALSE. If another telephony application has ownership of the telephone line, this status command  returns FALSE.

MCI_STATUS_TIME_FORMAT

The dwReturn field is set to the time format of the play/record media. This always returns MCI_FORMAT_MILLISECONDS.

MCI_TAM_STATUS_AUDIO_MUTE

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the mute status of the system microphone. When set to TRUE, the microphone (audio input of TAM_AUDIO) is disconnected from the telephone line, and any record operation in progress. When set to FALSE, the device operates normally.

MCI_TAM_STATUS_AUDIO_VOLUME

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_TAM_STATUS_AVGBYTESPERSEC

The dwReturn field is set to the actual average bytes per second of the current media record or play operation. This value is valid only when a desired rate has been set using MCI_TAM_SET_AVGBYTESPERSEC. See MCI_SET for details.

MCI_TAM_STATUS_BITSPERSAMPLE

The dwReturn field is set to the number of bits per sample (8 or 16) used for playing, recording, and saving, when using the PCM wave format.

MCI_TAM_STATUS_BITSPERSAMPLE is supported in version 3.1 and above of TAM drivers.

MCI_TAM_STATUS_CONNECT

The dwReturn field is set to the current device connections. Connected device flags are OR'ed together, and the result returned to the application. Connected devices are also the target of any play or record operations, thus for some operations, applications "connect" only a single device. The device flags are defined as follows:

·	MCI_TAM_AUDIO

·	MCI_TAM_HANDSET

·	MCI_TAM_PHONELINE

MCI_TAM_STATUS_CALL_FILTER	PL

The dwReturn field of the data structure identified by lParam2 is set to TRUE if the device is currently set to receive voice calls; otherwise it is set to FALSE.

MCI_TAM_STATUS_DIAL_FLASH_TIME	PL

The dwReturn field is set to the current flash time (in milliseconds) of the telephone flash option in the MCI_DIAL command.

MCI_TAM_STATUS_DIAL_FLASH_TIME is supported in version 2.2 and above of the TAM drivers.

MCI_TAM_STATUS_DIAL_PAUSE_TIME	PL

The dwReturn field is set to the current pause time (in milliseconds) that an embedded ',' character produces in the dial string.

MCI_TAM_STATUS_DIAL_WAIT_TIME	PL

The dwReturn field is set to the current time-out limit (in milliseconds) that an embedded 'w' character in the dial string  allows, waiting for a second dial tone.

MCI_TAM_STATUS_FORMATTAG

The dwReturn field is set to the format tag of the current device being recorded or played. The following formats are allowed:

·	WAVE_FORMAT_PCM (Supported in version 3.1 and above of the TAM drivers)

·	TAM_WAVE_FORMAT_CUSTOM

MCI_TAM_STATUS_HANDSET

The dwReturn field is set to the current status of the telephone handset. It is set to TRUE if the handset is off-hook; otherwise, it is set to FALSE.

MCI_TAM_STATUS_HANDSET_MUTE	MSG

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the mute status of the telephone handset. When set to TRUE, the audio input of the handset is disconnected from the telephone line, and any record operation in progress. When set to FALSE, the device operates normally.

MCI_TAM_STATUS_HANDSET_VOLUME	MSG

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_TAM_STATUS_HOOK	PL

The dwReturn field is set to the current hook status of the telephone line. It is set to TRUE if the phone is off-hook; otherwise, it is set to FALSE.

MCI_TAM_STATUS_LINE	PL

The dwReturn field is set to the current phone line status. The following status modes are defined:

·	MCI_PHONE_LINE_ONHOOK

·	MCI_PHONE_LINE_DIALTONE

·	MCI_PHONE_LINE_BUSY

·	MCI_PHONE_LINE_QUIET

·	MCI_PHONE_LINE_RINGTONE

·	MCI_PHONE_LINE_VOICE

·	MCI_PHONE_LINE_FAX

·	MCI_PHONE_LINE_MODEM

·	MCI_PHONE_LINE_UNKNOWN

MCI_TAM_STATUS_MAX_AUDIO_VOLUME

Some countries limit the maximum audio volume that the application can set.  To determine if the application is running in such a country, the application should issue MCI_STATUS for MCI_STATUS_WORLD_TRADE_SUPPORT. If the information returned from that call indicates LIMIT_MAX_VOLUME, the dwReturn field of this call is set to the maximum audio volume permitted in the country.  This support is added with driver version 3.3.

MCI_TAM_STATUS_MAX_FLASH_TIME	PL

The dwReturn field is set to the current maximum time between on-hook and off-hook that will be reported as a handset flash on a PHONE_EVENT_HANDSET_KEY event.

MCI_TAM_STATUS_MAX_GREETING_LEN

Some countries limit the maximum duration a greeting may be.  To determine if the application is running in such a country, the application should issue MCI_STATUS for MCI_STATUS_WORLDTRADE_SUPPORT. If the information returned from that call indicates LIMIT_GREETING_LENGTH, the dwReturn field of this call is set to the maximum greeting length, in seconds, permitted in the country.  It is up to the application to make sure the greeting doesn’t exceed this length. This support is added with driver version 3.3.

MCI_TAM_STATUS_MAX_GREETING_LEN_NO_REC

Some countries limit the maximum duration a greeting may be when no message is going to be recorded.  To determine if the application is running in such a country, the application should issue MCI_STATUS for MCI_STATUS_WORLDTRADE_SUPPORT. If the information returned from that call indicates LIMIT_GREETING_LENGTH_NO_RECORD, the dwReturn field of this call is set to the maximum permitted greeting length, in seconds, when no message is going to be recorded.  It is up to the application to make sure the greeting doesn’t exceed this length. This support is added with driver version 3.3.

MCI_TAM_STATUS_MAX_MIC_GAIN

The dwReturn field is set to the maximum permitted microphone gain, in decibels.  This supported is added with driver version 3.3.

MCI_TAM_STATUS_MAX_MSG_RETRIEVE_LEN

Some countries limit the maximum time between user inputs. This would require that the application gets input from the user at least every ‘n’ seconds.  To determine if the application is running in such a country, the application should issue MCI_STATUS for MCI_STATUS_WORLDTRADE_SUPPORT. If the information returned from that call indicates LIMIT_MAX_MSG_RETRIEVE_LENGTH, the dwReturn field of this call is set to the maximum length, in seconds, that messages can be played without prompting the user for input (e.g., DTMF keys).  It is up to the application to make sure greetings don’t exceed this length. This support is added with driver version 3.3.

MCI_TAM_STATUS_MAX_RECORD_LEN

Some countries limit the maximum duration of a message recorded from the phoneline.  To determine if the application is running in such a country, the application should issue MCI_STATUS for MCI_STATUS_WORLDTRADE_SUPPORT. If the information returned from that call indicates LIMIT_MESSAGE_RECORD_LENGTH, the dwReturn field of this call is set to the maximum length, in seconds, of a message recorded from the phoneline, permitted in the country.  It is up to the application to make sure the greeting doesn’t exceed this length. The simplest way to accomplish this is to specify MCI_TO on the MCI_RECORD, and use the value returned from this call as the MCI_TO value. This support is added with driver version 3.3.

MCI_TAM_STATUS_MICROPHONE_GAIN

The dwReturn field is set to the current microphone gain, in decibels.

MCI_TAM_STATUS_MIN_FLASH_TIME	PL

The dwReturn field is set to the current minimum time between on-hook and off-hook that will be reported as a handset flash on a PHONE_EVENT_HANDSET_KEY event.

MCI_TAM_STATUS_QUALITY

The dwReturn field is set to the current telephone device play/record quality level. Expected range of quality is from 0 (lowest quality) to 7 (highest quality).

MCI_TAM_STATUS_QUIET_DURATION	PL

The dwReturn field is set to the current value for the continuous phone line quiet time in seconds before an application will get the first MM_MCIEVENT message specifying CALL_PROGRESS_QUIET.  Zero indicates the application will not get the CALL_PROGRESS_QUIET interrupt returned.  After receiving the first  CALL_PROGRESS_QUIET, the application continues receiving this message every second until the call terminates.

MCI_TAM_STATUS_QUIET_DURATION is not supported in current driver.

MCI_TAM_STATUS_RING_COUNT	PL

The dwReturn field is set to a constant specifying the ring count at which the device answers the telephone. The driver answers on the shortest ring count request of all active applications, so this value might not match the value specified in MCI_SET.

MCI_TAM_STATUS_SAMPLESPERSEC

The dwReturn field is set to the number of samples per second used for playing, recording, and saving, when using the PCM wave format.

MCI_TAM_STATUS_SAMPLESPERSEC is supported in version 3.1 and above of the TAM drivers.

MCI_TAM_STATUS_SPEED	MSG

The dwReturn field is set to the device speed factor of the current device. See MCI_TAM_SET_SPEED for details.

MCI_TAM_STATUS_WORLDTRADE_SUPPORT

The dwReturn field is set to a binary encoded set of values indicating restrictions that are in effect for the current country.  Some of the bit settings require the application to make a subsequent MCI_STATUS call to determine a maximum value. This support is added with driver version 3.3. The defined bits include:

·	GAIN_CHANGE_NOT_ALLOWED is set TRUE if the application is not permitted to change the microphone gain.

·	GAIN_CHANGE_NOT_ALLOWED_OFFHOOK is set TRUE if the application is not permitted to change the microphone gain when the phone is off hook.

·	LIMIT_MAX_VOLUME is set TRUE if the maximum speaker volume is limited.  See MCI_STATUS for  MCI_TAM_STATUS_MAX_AUDIO_VOLUME for related information.

·	LIMIT_GREETING_LENGTH is set TRUE if the greeting length is limited.  See MCI_STATUS for MCI_TAM_STATUS_MAX_GREETING_LEN for related

information.

·	LIMIT_GREETING_LENGTH_NO_RECORD is set TRUE if the greeting length is limited when no message will be recorded.  See MCI_STATUS for MCI_TAM_STATUS_MAX_GREETING_LEN_NO_REC for related information.

·	DISALLOW_GREETING_WITH_NO_RECORD is set TRUE if informational greetings are not permitted.

·	LIMIT_MESSAGE_RECORD_LENGTH is set TRUE if the length of messages recorded from the phone line is limited.  See MCI_STATUS for MCI_TAM_STATUS_MAX_RECORD_LEN for related information.

·	REMOTE_GREETING_RECORD_REVIEW is set TRUE if the country requires the application to play back a remotely recorded greeting before the new greeting goes into effect.

·	LIMIT_MAX_MSG_RETRIEVE_LENGTH is set TRUE if the country limits the maximum time between user input.  This would require that the application get input from the user every ‘n’ seconds.  See MCI_STATUS for MCI_TAM_STATUS_MAX_MSG_RETRIEVE_LEN for related information.

·	NEVER_ANSWER_SILENT is set TRUE if the phone can never be answered with silence.

·	TAM_NOT_ALLOWED_IN_COUNTRY is set TRUE if the telephone answering machine functions are not permitted in the country.  If this is the case the PL application will get a bad return code on MCI_OPEN.  However the MSG application can query this information.

·	SPK_PHONE_NOT_ALLOWED_IN_COUNTRY is set TRUE if connecting to speaker phone is not permitted.

·	AUTODISCRIM_TAM_NOT_ALLOWED is set TRUE if the  	automatic call discrimination of voice calls is not permitted.

·	AUTODISCRIM_FAX_NOT_ALLOWED is set TRUE if the  	automatic call discrimination of FAX calls is not permitted.

·	AUTODISCRIM_MODEM_NOT_ALLOWED is set TRUE if the  	automatic call discrimination of MODEM calls is not permitted.

This support is added with driver version 3.4.

·	PULSE_DIAL_NOT_ALLOWED is set TRUE if pulse dialing is

not supported.

·	DTMF_DIAL_NOT_ALLOWED is set TRUE if DTMF dialing is

not supported.

·	BUSYTONE_DETECT_NOT_VALID is set TRUE if busy tone	detection is not available in the country.

·	BUSYTONE_DETECT_REQUIRED is set TRUE if busy tone	detection is required in country.

·	DIALTONE_DETECT_NOT_VALID is set TRUE if dial tone	detection is not available in the country.

·	DIALTONE_DETECT_REQUIRED is set TRUE if dial tone	detection is required in country.

- OFFHOOK_NOT_ALLOWED_HANDSET_UP if the application is not permitted to have the phone electronically off hook (SET HOOK TRUE) when the handset is up.

MCI_TAM_STATUS_COUNTRY_CODE

The dwReturn field is set to the current country code.  This can be used by applications that must change the looks of the user interface for different countries like a French keypad in France. This support is added with driver version 3.4. The following table shows the codes assigned to each country:

| COUNTRY | CODE |  | COUNTRY | CODE |  | COUNTRY | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| USA/Canada | 1 |  | Australia | 14 |  | Norway | 27 |
| Belgium | 2 |  | Austria | 15 |  | Denmark | 28 |
| Hong Kong | 3 |  | Mexico | 16 |  | France | 29 |
| Singapore | 4 |  | South Africa | 17 |  | Netherlands | 30 |
| New Zealand | 5 |  | Chile | 18 |  | U. K. | 31 |
| Japan | 6 |  | Switzerland | 19 |  | Sweden | 32 |
| Portugal | 7 |  | Germany | 20 |  | Italy | 33 |
| Ireland | 8 |  | Brazil | 21 |  | Finland | 34 |
| Generic | 9 |  | Russia | 22 |  | Thailand | 35 |
| Spain | 10 |  | Yugoslavia | 23 |  | Korea | 36 |
| Greece | 11 |  | Hungary | 24 |  | Malaysia | 37 |
| Israel | 12 |  | Czechrepublic | 25 |  | PRC | 38 |
| Taiwan | 13 |  | Luxembourg | 26 |  | Slovakia | 39 |
|  |  | TABLE 7-4: Country Codes |  |  |  |  |  |

MCI_TAM_STATUS_AUTO_ANSWER_MIN_RINGS

The dwReturn field contains the minimum number of rings that can be set in MCI_TAM_SET_RING_COUNT. If the value is 'FFFF'x then there is no min in that country. This support is added with driver version 3.4.

MCI_TAM_STATUS_AUTO_ANSWER_MAX_RINGS

The dwReturn field contains the maximum number of rings can be set in MCI_TAM_SET_RING_COUNT.  If the value is 'FFFF'x then there is no limit in that country. This support is added with driver version 3.4.

MCI_TAM_STATUS_MAX_CALL_RETRIES

The dwReturn field contains the maximum number of unsuccessful retries allowed.  If the value is 'FFFF'x there is no max in that country. This support is added with driver version 3.4.

MCI_TAM_STATUS_MIN_CALL_RETRY_TIME

The dwReturn field contains the minimum time allowed between retries. If the value is 'FFFF'x then there is no min in that country. This support is added with driver version 3.4.

LPMCI_STATUS_PARMS   lParam2

Specifies a far pointer to the following MCI_STATUS_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwReturn;

DWORD		dwItem;

} MCI_STATUS_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_STOP

This command message stops an MCI_PLAY or MCI_RECORD command in operation.

Parameters	DWORD    lParam1

The following flags apply to the TAM device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_STOP_REMOVE_DTMF

Specifies that if the MCI_STOP is stopping a record operation, and the record has recorded a DTMF key, all information recorded after the first DTMF key was pressed will be removed from the recorded message.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.