| V4P895GRN (4x72pin) MB CPU Jump Setting selection CPU selection (JP8, JP9, JP15, JP16, JP17, JP18, JP19, JP20, JP23, JP26, JP27, JP28, JP29, JP30, JP31, JP32, JP36) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CPU TYPE | 486SX | 486DX | SL-486SX | SL-486DX | INTEL 486DX4 | AMD 3.3V CPU |  |
| JP8 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |  |
| JP9 | OPEN | OPEN | OPEN | OPEN | OPEN | OPEN |  |
| JP15 | OPEN | OPEN | OPEN | OPEN | OPEN | OPEN |  |
| JP16 | OPEN | OPEN | OPEN | OPEN | OPEN | OPEN |  |
| JP17 | OPEN | OPEN | OPEN | OPEN | OPEN | OPEN |  |
| JP18 | OPEN | OPEN | OPEN | OPEN | OPEN | 1-2 : 3X CLK 2-3 : 2X CLK |  |
| JP19 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |  |
| JP20 | OPEN | OPEN | OPEN | OPEN | OPEN: 3X CLK 1-2 : 2X CLK 2-3 : 2.5 CLK | OPEN |  |
| JP23 | 1-2 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 |  |
| JP26 | OPEN | OPEN | CLOSE | CLOSE | CLOSE | OPEN |  |
| JP27 | OPEN | OPEN | CLOSE | CLOSE | CLOSE | OPEN |  |
| JP28 | OPEN | OPEN | OPEN | OPEN | OPEN | OPEN |  |
| JP29 | OPEN | 1-2 | OPEN | 1-2 | 1-2 | 1-2 |  |
| JP30 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |  |
| JP31 | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 | 1-2 |  |
| JP32 | OPEN | 2-3 | OPEN | 2-3 | 2-3 | 2-3 |  |
| JP36 | OPEN | OPEN | OPEN | OPEN | CLOSE | OPEN |  |

| CPU TYPE | AMD 3.3V Enhance CPU | AMD 3.3V X5-133 CPU | P24D | CYRIX 486 DX CPU CYRIX 5X86 3.3V CPU CYRIX M1 3.3V CPU |
| --- | --- | --- | --- | --- |
| JP8 | 2-3 | 2-3 | 2-3 | 5V CPU : 2-3 3V CPU : 1-2 |
| JP9 | OPEN | OPEN | OPEN | OPEN |
| JP15 | CLOSE | CLOSE | CLOSE | OPEN |
| JP16 | CLOSE | CLOSE | CLOSE | OPEN |
| JP17 | 2-3 | 2-3 | 2-3 | 1-2 |
| JP18 | 1-2 : WB 2-3 : WT | 1-2 | 1-2 : WB 2-3 : WT | OPEN |
| JP19 | 1-2 | 1-2 | 1-2 | 2-3 |
| JP20 | OPEN | 1-2 : 4X CLK | OPEN | OPEN |
| JP23 | 2-3 | 2-3 | 2-3 | 1-2 |
| JP26 | CLOSE | CLOSE | CLOSE | OPEN |
| JP27 | CLOSE | CLOSE | CLOSE | OPEN |
| JP28 | OPEN | OPEN | OPEN | 1-2 : M7 1X CLK 2-3 : M7 2X CLK |
| JP29 | 1-2 | 1-2 | 1-2 | 1-2 |
| JP30 | 1-2 | 1-2 | 1-2 | 1-2 |
| JP31 | 1-2 | 1-2 | 1-2 | 1-2 |
| JP32 | 2-3 | 2-3 | 2-3 | 2-3 |
| JP36 | OPEN | OPEN | OPEN | OPEN |

"WB" stand for Write back

"WT" stand for Write Through