| Field Note: 232 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Apocalypse 3D - Getting Started

Date Created: 	July 29, 2026

Product: 	Apocalypse 3D

Introduction

This file contains troubleshooting information for Apocalypse 3D.  This Field Note should be read in conjunction with the ReadMe supplied with the VideoLogic Driver CD.

I’ve installed the drivers - I want to quickly check that it is running!

Once the drivers are installed you can check that your system is ready by running applications designed to use each of the two drivers.  Prior to running a sample application it is a good idea to verify the following :

- The graphics mode is set to 16 bits per pixel or greater.

- The graphics drivers are DirectX III compatible.

- The graphics card has 2MB or more of memory

- DirectX III has been installed

Once you are satisfied that the system is ready, try one of the PowerSGL applications provided on the CD such as Tower4.  Note, that the application Antworld requires a joystick and the application Tower5 requires a graphics mode greater than 640x480.

Secondly, run any of the D3D applications provided with your CD.  Note that if the Apocalypse 3D card is not functioning in your system, you should still see the application running.  This is because those sample applications can also run using software emulation to simulate a 3D accelerator card.  To verify your Apocalypse 3D card is functioning, go to the ‘File’ menu of your D3D application. The entry ‘Direct3D HAL’ will be visible and have a tick next to it if your card is working. If the ‘Direct3D HAL’ text is missing and instead ‘Ramp Emulation’ is ticked, then your Apocalypse 3D card is not functioning.

Games graphics display a ‘halo’ around objects

This problem can be rectified by running the D3D Setup utility, as described in section 5 of the Read Me. For other graphics problems, access our web site where the latest version of our D3D drivers registry update can be found.

With the D3D demos I am not given the D3D Hal option

Ensure that you have the DirectX III graphics drivers installed, and that your PC is in a display mode greater or equal to 16 bits per pixel.

The application display is corrupted

Your PC may not be fully DirectDraw compliant. This can be tested using the DirectDraw Compliance utility provided on the drivers disk.

The application looks fuzzy, with lines appearing alongside the active window

The resolution of the window in which the application is running is too high, or the refresh rate of the screen is too high. Lower the resolution or the refresh rate in Display Properties.

The Start menu and Windows menus contain corrupted text, or text is missing

After playing a game in 8-bit display mode, the PC has changed from 8-bit to 16-bit display mode automatically, which the display driver may not be able to support. Restart the PC.

To avoid this problem in future, your PC should be in a 16-bit display mode before playing a game.

When running an application that uses PowerSGL or Direct3D, the system freezes or restarts

Apocalypse 3D may be installed in a PCI bus slot that does not have bus-mastering capability.

Note: Apocalypse 3D cannot work in systems where the PCI bus speed is set to  25MHz (e.g. Intel Pentium 75 and Cyrix 6x86 P120)

When running an application that uses PowerSGL the system restarts

The display driver may not support buffer flipping. To support flipping the amount of free off-screen memory must be greater than the current on-screen memory. A lower screen resolution may free up sufficient memory, however, due to other graphics memory requirements (e.g. hardware cursors), 1 MB cards may not support flipping even in a resolution of 640x480 pixels with 65 thousand colors. This is why we recommend a minimum graphics memory of 2 MB.

The Use Hardware Accelerator option within an application is greyed out

Solution 1:	The application is currently set to run in a graphics resolution that cannot be accelerated by Apocalypse 3D. Changing the application’s graphics resolution setting to a suitable mode will make the check-box available.

Solution 2:	The Apocalypse 3D board is not being recognised by the system. Check that it is in System Properties, check BIOS, IRQ allocation etc.

Other Relevant Field Notes :

If any of the system features listed below are relevant to your system then please refer to the appropriate Field Note.

System Feature						Field Note

Award BIOS							FN230

S3 968 or S3 868 graphics card				FN230

(such as GrafixStar700, Diamond Stealth 64 VRAM etc..)

Matrox graphics cards						FN231

VideoLogic Support Team

July, 26

Apocalypse 3D has two sets of drivers incorporated into one installation - D3D (incorporating PowerSGL).  PowerSGL is VideoLogic's proprietary API for use with titles specifically designed for Apocalypse 3D.  D3D Drivers allow you play games designed to use Microsoft's D3D API.