PL26CPQ.COM

INSTRUCTIONS FOR USE

PL26CPQ.COM is a file patching utility for use with the Compaq Deskpro

486/50M or 486/66M and any program that contains the Phar Lap

386|DOS-Extender Version 2.6.

The problem manifests itself as hanging, rebooting, or displaying

various error messages when attempting to run the affected program.  If

memory cache is turned off (MODE MEM OFF) the program runs without any

problems.  Since PL26CPQ.COM patches the file on disk it only needs to

be run once for any particular program.

PL26CPQ must be given a command line argument of the file name of the

program to be patched.  A drive and/or path can be included with the

program name.  For example, to patch the "NEWDX"ed version of AutoCAD

release 11 - assuming PL26CPQ is in the same subdirectory as ACAD.EXE

enter at the DOS command prompt:

PL26CPQ ACAD.EXE

If PL26CPQ patches the ACAD.EXE file it will display:

Patch successfully applied

If PL26CPQ does not patch the file, it will display the reason - either

the file is not Phar Lap based or the patch is not required.

PL26CPQ.COM supersedes ACAD.COM (SP0313) and SHADE.COM (SP0347).