OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	May 7, 1995

SUBJECT:	Morrison32 Bios Release 3.0

About This Release

o	BIOS 1.00.03.BT0 Release 3.0 Morrison32

Features/Errata Fixed In This Release

Release 3.0

o	Included new video bios 1.03.08.12 that fixes the problem where palette snooping was always turned off.

o	Fixed security hotkey, the key entered by the user in setup was being reprogrammed in the keyboard controller.  The bios was writing the user entered value then later writing the scan code for the back space key.

Release 2.0

o 	Fixed problems with S3 Trio 32 Video failures via reduction in Video Bios Clock rate reduction from 60Mhz to 57Mhz.  Also corrected font issues.  Video bios revision 1.03.08.09-57Mhz.

o	Fixed problem with ISA Shared Memory Setup options which was introduced between Beta 2.01 and Beta 2.02.  Now back to Beta 2.01 and Release 1.0 operation.  Failure was characterized by a failure of PCI video (with video error beep) when one would set the ISA Shared Memory option to start at C8000h.

o	Fixed problem introduced since Release 1.0 which would cause Protected Mode applications/Operating Systems to have a General Protection Fault when modifying the Device Node  configurations for onboard Parallel Port or onboard PCI IDE.

o	Fixed problem revolving around Bios Data Area offset 6Ch (dword - 32 bit value) aka Timer Count value.  Symptom of problem fixed:  “incorrect updating of Timer Count value in Bios Data Area occurs during Sleep/Power-Down when running Windows 95”.  Fixed by utilizing Real Time Clock AND IRQ 0 (was previously only IRQ 0) as conditions for updating Timer Count value in Bios Data Area.

o	Added enhancement to APM code to allow audio feedback (beeping) which indicates APM code detection of:

a)	Sleep/Resume/Soft Power Off button press (single beep),

b)	Soft Power Off via 2 second hold of Sleep/Resume/Soft Power Off button (two decending beeps).

Beta 2.02

o	Adjusted Language Set Support to handle Single Language Sets only (U.S. English only, French only, Spanish only, Italian only, or German only).  Previous Releases supported Dual Language Sets (U.S. English and one of the following languages:  French, Spanish, Italian, or German).  However, due to size issues only Single Language Sets can now be supported.

o	Fixed PCI code dealing with devices Disabled (DCS #2966).

o	Fixed PCI Bridge I/O Space allocation issues.

o	Adjusted code to always dispatch APM code reguardless of Enable or Disabled via SETUP option.

o	Adjusted Motherboard Device Node to change the DMA capability of the Onboard Floppy from Bus Mastering to Non-Bus Mastering.

o	Adjusted Motherboard Device Node to change the Serial Port Device Identifier from a Non-FIFO to a FIFO type device.

o	Fixed Bios Error Display code to Not Wait for User Input if no Keyboard was detected.

o	Fixed Bios Error Display code to Not Wait for User Input if no Video was detected (DCS #3401).

o	Adjusted SETUP to allow IRQ 5 or 7 to be chosen for I/O 378h for parallel port.

o	Adjusted parallel port mode choices to support Compatible, Bi-directional, ECP, & EPP.

o	Fixed Bios Error Display code to Not Display Resource Conflict Message when no PS/2 Mouse and add-in card using IRQ 12.

o	Fixed OS/2 trap problem related to APM being Enabled/Disabled in SETUP.

o	Fixed issue related to stack handling during compatible Shutdown 9.

o	Adjusted Resource Auto-configuration order for Windows 95 so that Motherboard Resources will be allocated before PCI resources.  This has been implemented via a SETUP option.

o	Adjusted PCI Resource allocation related to Video IRQ assignment.  We can now, on a product by product basis, decide to not assign IRQ’s to PCI Video devices.

o	Fixed the programming of the Cache Line Size and Latency Timer for Bridges.

o	Fixed Motherboard Device Node Attributes to indicate “device can only be configured at runtime” from “device can be configured at runtime”.  This fixes issue with WINICU R1.41 which thought the motherboard devices could be reconfigured for a next boot situation but the System Bios doesn’t support this.

o	Fixed SETUP code to make sure one interrupt was preserved for PCI usage.

o	Fixed IDE timing programming so that when have two different mode drives ona single channel the slower mode will get programmed for that channel.

o	Fixed SETUP code for display of IRQ 5 and IRQ 7 in Plug and Play screen.

o	Fixed issue where PIIX Bus Mastering IDE was not being enabled.

o	Fixed APM code issues related to missing interrupts.

o	Fixed APM code for alarm events.

o	Adjusted APM code so front panel switch being held for 3 seconds will be interpreted as a soft power off.

o	Fixed PIIX IDE Master Latency Timer programming per SETUP (DCS #3354).

o	Fixed Motherboard Device Node for IDE reporting it is always enabled even if not (DCS #2759).

o    	Added new Crystal Audio micro-code.  Fixes problems of hangs at boot time and issues with Magic Carpet Software.

Beta 2.01

o	Fixed 32 bit PCI Set Irq to check for valid PCI Busses.  PCI Local Bus Specification Revision 2.1 change.

o	Added Product Information Storage and Retrieval features.

o	Fixed PCI and ISA PnP auto-configuration code so that a Configuration Error is flagged and the device is not disabled (DSC #3394).

o	Adjusted ISA PnP auto-configuration code so that back-to-back Isolation is not performed.  This helps certain ISA PnP cards which cannot handle this.

o	Fixed IDE auto-configuration code so that an empty IDE channel (an IDE channel without any drives connected to it) woul not hang the System Bios at POST-Time.

o	Fixed jump to wrong label in ISA PnP auto-configuration code.

o	Fixed PCIDIAG V1.2 hang and extraneous PCI.EXE entries.

o	Fixed Int 13h, Function 0Ch which was returning invalid value in AH.

o	Adjusted Message Server to handle Languages greater than 64k in size.

o	Adjusted ATA code to issue a second reset after ATA initialization to fix problem seen in some IDE CD-ROM drives not being accessible after boot.

o	Fixed code which handles freeform fields in SETUP to not update a field if invalid entry received.

o	Adjusted Motherboard Device Nodes to ignore the Checksum (per the “Plug and Play BIOS Specification” Version 1.0A) for Set System Node.

o    	Added new Crystal Audio micro-code.

Release 1.0

o     Removed Floppy Auto-Configuration from setup.  Only manual floppy configuration now supported.

o     Removed Soft Power Off setup option now supported in hardware.

o     Fixed ESCD to free up previously used PCI/PNP slots entries so they can be used again..

o     Fixed user password failure with seven characters entered for password.  Failure scenerio involved “Unattended Start” and hot key not functioning properly.

Beta 1.04

o     Fixed 32 bit -protect mode interface for PCI/PNP calls whihc manifested itself in Windows ICU R1.4 not functioning properly.

o     Adjusted memory error beep code for cleaner sound if have memory error.

Beta 1.03

o     Added in latest S3 Trio 64 Video Bios.

o     Fixed problem where offboard floppy wasn’t functioning properly.

o     Added custom IRQ/DMA for better resource allocation when audio is stuffed.

o     Added setup field to replace Cirrus 5434 PCI Burst detection and make it a generic setup option for video cards who can’t handle PCI Bursting..

Beta 1.02

o     Fixed problem with latest ICU.  Need to use the following ICU until a new version is released (CM 1.24, Windows ICU R1.3).

o     Fixed problem with floppy where if disabled not reflected in Bios Data Area.

o     Enabled PCI Special Cycles in PIIX so that Triple Fault will cause reset.

o     Added Bridge Record for DTEK Compliance.

o     Fixed issue where Bios would improperly handle 2M byte SIMMs as though they were a supported part.  2M byte SIMMs are not supported and now will get error beep if used or will be ignored if used in combination with valid/supported SIMMs.

o     Added support to display speed of 120M hz processor in setup if used.

o     Updated DRAM timings per 1/4/95 memo from H/W.

Beta 1.01

o     Fixed problem where “Entering Setup” message was delayed after pressing the F1 key.

o     Fixed problem where ICU reassigned ports over the read data port.

o     Code updated to always scan all devices rather that use last output device. This allows an ISA card to override an on-board video.

o     Update to parallel port manual mode code.

o     IRQ 15 no longer used for PCI IRQ assignments.

o     Updated 8051 uCode from Crystal.

o     PIIX DMA channel programming left at default.  This assumes the “4-wire” H/W update has been done to route the Crystal DMA channels directly to the ISA bus.

o     Added code to disable pci bursting if Cirrus 5434 video controller is present and active.

o     Removed EDO auto/manual detection mode code.

o     Updated dram timings per 12/10/94 memo from H/W.

o     Fixed parallel port ECP support.

o     Updated floppy code so manual mode doesn't do a recalibrate if disabled.

o     Added code to generate error if memory requested below 1MB and not on bus 0.

o     Fixed Floppy Drive B: issues for 1.2MB 5.25” media failures.

o     Adjusted so that IRQ 12 can be used if no PS/2 Mouse installed.

o     Fixed Static Resource Conflict for ISA Cards w/ ICU.

o     Fixed Floppy issues related to 2.88M formats & drive B default type.

o     Resolved issues related to PCI Confidence test failures for PCIPOST State 23h.

o     Fixed Int 15h Function 87h interface.

o     System BIOS Caching Disabled to fix OS/2 Boot issues.

o     APM updated for new ATA IDE Hard Disk Table.

o     Cache Enable/Disable Algortihm fixed.

o     Revised Algorithm used to access PCI Configuration Space.

o     Memory Sizing fixed.

Features Included In This Release

Release 3.0

o	None.

Release 2.0

o	Added enhancement to APM code to allow audio feedback (beeping) which indicates APM code detection of:

a)	Sleep/Resume/Soft Power Off button press (single beep),

b)	Soft Power Off via 2 second hold of Sleep/Resume/Soft Power Off button (two decending beeps).

Beta 2.02

o	Adjusted Language Set Support to handle Single Language Sets only (U.S. English only, French only, Spanish only, Italian only, or German only).  Previous Releases supported Dual Language Sets (U.S. English and one of the following languages:  French, Spanish, Italian, or German).  However, due to size issues only Single Language Sets can now be supported.

o	Adjusted code to always dispatch APM code reguardless of Enable or Disabled via SETUP option.

o	Adjusted Motherboard Device Node to change the DMA capability of the Onboard Floppy from Bus Mastering to Non-Bus Mastering.

o	Adjusted Motherboard Device Node to change the Serial Port Device Identifier from a Non-FIFO to a FIFO type device.

o	Adjusted SETUP to allow IRQ 5 or 7 to be chosen for I/O 378h for parallel port.

o	Adjusted parallel port mode choices to support Compatible, Bi-directional, ECP, & EPP.

o	Adjusted Resource Auto-configuration order for Windows 95 so that Motherboard Resources will be allocated before PCI resources.  This has been implemented via a SETUP option.

o	Adjusted PCI Resource allocation related to Video IRQ assignment.  We can now, on a product by product basis, decide to not assign IRQ’s to PCI Video devices.

o	Adjusted APM code so front panel switch being held for 3 seconds will be interpreted as a soft power off.

Beta 2.01

o	Added Product Information Storage and Retrieval features.

o	Adjusted ISA PnP auto-configuration code so that back-to-back Isolation is not performed.  This helps certain ISA PnP cards which cannot handle this.

o	Adjusted Message Server to handle Languages greater than 64k in size.

o	Adjusted ATA code to issue a second reset after ATA initialization to fix problem seen in some IDE CD-ROM drives not being accessible after boot.

o	Adjusted Motherboard Device Nodes to ignore the Checksum (per the “Plug and Play BIOS Specification” Version 1.0A) for Set System Node.

o    	Added new Crystal Audio micro-code.

Release 1.0

o     None.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     None.

Beta 1.01

o     Added new ESCD PCI Bridge support

o     Added support to Boot With PnP OS.

o     Floppy Auto Configuration adjusted to default to 1.44M not 2.88M when no media installed.

o     Floppy Auto Configuration algorithm streamlined for efficiency.

o     Enhanced ESCD support..

o     IDE ATA interface code added.

o	PCI Bridge Support.

o	Keyboard/Mouse Swap Support.

o	Phase III - Biosä.

o	POSThasteä.

o	National Semiconductor PC87306 Super I/O support

o	AMI Keyboard Controller Support

o	Triton Chipset Support.

o	Mode 4 IDE Support.

o	Static Keyboard/Mouse Support (does not support swapping).

o	Chicago Devnode Support.

o	Chicago APM Support.

Setup Changes

Release 3.0

o	None.

Release 2.0

o	Fixed problem with ISA Shared Memory Setup options which was introduced between Beta 2.01 and Beta 2.02.  Now back to Beta 2.01 and Release 1.0 operation.  Failure was characterized by a failure of PCI video (with video error beep) when one would set the ISA Shared Memory option to start at C8000h.

Beta 2.02

o	Adjusted Language Set Support to handle Single Language Sets only (U.S. English only, French only, Spanish only, Italian only, or German only).  Previous Releases supported Dual Language Sets (U.S. English and one of the following languages:  French, Spanish, Italian, or German).  However, due to size issues only Single Language Sets can now be supported.

o	Adjusted SETUP to allow IRQ 5 or 7 to be chosen for I/O 378h for parallel port.

o	Adjusted parallel port mode choices to support Compatible, Bi-directional, ECP, & EPP.

o	Adjusted Resource Auto-configuration order for Windows 95 so that Motherboard Resources will be allocated before PCI resources.  This has been implemented via a SETUP option.

o	Fixed SETUP code to make sure one interrupt was preserved for PCI usage.

o	Fixed SETUP code for display of IRQ 5 and IRQ 7 in Plug and Play screen.

o	Fixed PIIX IDE Master Latency Timer programming per SETUP (DCS #3354).

Beta 2.01

o	Fixed code which handles freeform fields in SETUP to not update a field if invalid entry received.

Release 1.0

o     Removed Floppy Auto-Configuration from setup.  Only manual floppy configuration now supported.

o     Removed Soft Power Off setup option now supported in hardware.

Beta 1.04

o     Fixed Hard Disk field for Fast Programmed I/O Modes.  Wasn’t being evaluated previously..

Beta 1.03

o     Added PCI Burst Enable/Disable Option to allow certain video cards to function without glitches.

Beta 1.02

o     None.

Beta 1.01

o     If a User password and an Administrative password are installed, and Setup is entered via the User password, the “Unattended Start” and “Security Hot Key” entries are accessible.

o     Removed Setup option for SIMM auto/manual detection (always auto).

o     Removed Setup option for IDE prefetch buffer.

o     “Boot with PnP OS” entry appears only if the “Configuration Mode” entry is set to “Use ICU”.

o     “ISA Shared Memory Size” entry now includes options for 80KB and 96KB.

o     Removed  “C0000h” and “C4000h” options from “ISA Shared Memory Base Address” entry.

o     Adjusted so that IRQ 12 will be displayed if no PS/2 Mouse installed.

o     Removed ISA IDE DMA option from hard disk setup pages.

o     Added Soft Power Off option to setup.

o     Added Boot With PnP OS option to setup.

o     Hard Disk Translation Mode Changed to default of “Auto Detected” from “Standard CHS”.

o     Adjusted Date format in Main Screen to Month Date Year, was Year Date Month.

o     APM Enabled by default.

o	Removed DRAM Timings option.

o	Removed Infra-Red options except enable/disable.

o	Added 3-Mode/Auto-Configure Floppy choices.

o	Changed Palette Snooping default value to Disabled.

o	New, more friendly, user interface.  Layered interface functions more like standard applications with better screen layout for easy viewing.  Like functions are grouped onto screens or menus for easier configuration.

o	On-line help available for most setup fields.

o	Administrative password function allows all but Time/Date and User Password selections to be modified only by the system administrator.

o	New screen layout allows for more descriptive field text.

o	New screen layout permits field and option text to be translated without having to use cryptic abbreviations.

o	Setup includes support for DRAM Timmings Selection Option.

o	Setup includes support for SIMM Type Overide.  EDO SIMMs or Fast Paged SIMMs.

Password Changes

Release 3.0

o	Security hotkey fixed.

Release 2.0

o	None.

Beta 2.02

o	None.

Beta 2.01

o     None.

Release 1.0

o     Fixed user password failure with seven characters entered for password.  Failure scenerio involved “Unattended Start” and hot key not functioning properly.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     None.

Beta 1.01

o	Both User and Administrative passwords are available.

o	Passwords have been extended to seven(7) characters instead of the current six(6).

Language Support

Release 3.0

o	None.

Release 2.0

o	None.

Beta 2.02

o	Adjusted Language Set Support to handle Single Language Sets only (U.S. English only, French only, Spanish only, Italian only, or German only).  Previous Releases supported Dual Language Sets (U.S. English and one of the following languages:  French, Spanish, Italian, or German).  However, due to size issues only Single Language Sets can now be supported.

Beta 2.01

o	Adjusted Message Server to handle Languages greater than 64k in size.

Release 1.0

o     None.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     None.

Beta 1.01

o	Multi-language support is now available.

o	OEMs and users will be able to flash language sets into the BIOS and configure which language is used through Setup.  The current plas is for language pairs consisting of: English(US)/German(GR), English(US)/Italian(IT), English(US)/French(FR), and English(US)/Spanish(SP).

o	Depending upon the size of the FLASH device used for a particular product, up to 32 languages may be present at one time in the BIOS.

Hard Disk Changes

Release 3.0

o	None.

Release 2.0

o	None.

Beta 2.02

o	Fixed IDE timing programming so that when have two different mode drives ona single channel the slower mode will get programmed for that channel.

o	Fixed issue where PIIX Bus Mastering IDE was not being enabled.

Beta 2.01

o	Fixed IDE auto-configuration code so that an empty IDE channel (an IDE channel without any drives connected to it) woul not hang the System Bios at POST-Time.

o	Fixed Int 13h, Function 0Ch which was returning invalid value in AH.

o	Adjusted ATA code to issue a second reset after ATA initialization to fix problem seen in some IDE CD-ROM drives not being accessible after boot.

Release 1.0

o     None.

Beta 1.04

o     Fixed Hard Disk field for Fast Programmed I/O Modes.  Wasn’t being evaluated previously..

Beta 1.03

o     None.

Beta 1.02

o     Fixed issue of system hang when no onboard IDE hard disks are installed in the system.

Beta 1.01

o     IDE ATA Support added.

o	User definable parameters are available on a per drive basis.  This means each of the four drives may have unique user defined parameters.

o	The DMA selection (Auto-Configured or Disabled) may be set on a per drive basis.  (The ability to select the DMA transfer mode has been removed.)

o	The Sector/Block selection (Auto-Configured, 8 Sectors/Block, 4 Sectors/Block, or Disabled) for multiple sector transfers may be set on a per drive basis.  (The ability for finer resolution of Sector/Block size has been removed.)

o	The Translation selection (Auto-Configured, Logical Block Addressing, Extended Cylinder/Head/Sector or Standard Cylinder/Head/Sector) may be set on a per drive basis.

o	A Hard Disk Pre-Delay has been added to the Boot Options screen.  This option sets a fixed amount of time to wait in POST before attempting to initialize any of the hard disks.  This delay occurs only once before calling the hard disk initialization code.  Because POST is so fast on this version of BIOS, many drives have not finished their power on self-test routines by the time the BIOS reaches its hard disk initialization code.  For certain drives, if the BIOS attempts to query the drive before it has finished its self-test, the drive may hang.  In an attempt to accommodate a user with one of these drives this option has been made available.  (This option is available on some current products.)

Plug and Play (Windows 95, etc.)

Release 3.0

o	None.

Release 2.0

o	Fixed problem with ISA Shared Memory Setup options which was introduced between Beta 2.01 and Beta 2.02.  Now back to Beta 2.01 and Release 1.0 operation.  Failure was characterized by a failure of PCI video (with video error beep) when one would set the ISA Shared Memory option to start at C8000h.

o	Fixed problem introduced since Release 1.0 which would cause Protected Mode applications/Operating Systems to have a General Protection Fault when modifying the Device Node  configurations for onboard Parallel Port or onboard PCI IDE.

o	Fixed problem revolving around Bios Data Area offset 6Ch (dword - 32 bit value) aka Timer Count value.  Symptom of problem fixed:  “incorrect updating of Timer Count value in Bios Data Area occurs during Sleep/Power-Down when running Windows 95”.  Fixed by utilizing Real Time Clock AND IRQ 0 (was previously only IRQ 0) as conditions for updating Timer Count value in Bios Data Area.

Beta 2.02

o	Fixed PCI code dealing with devices Disabled (DCS #2966).

o	Adjusted Motherboard Device Nodes to ignore the Checksum (per the “Plug and Play BIOS Specification” Version 1.0A) for Set System Node.

o	Fixed PCI Bridge I/O Space allocation issues.

o	Adjusted Motherboard Device Node to change the DMA capability of the Onboard Floppy from Bus Mastering to Non-Bus Mastering.

o	Adjusted Motherboard Device Node to change the Serial Port Device Identifier from a Non-FIFO to a FIFO type device.

o	Fixed Bios Error Display code to Not Display Resource Conflict Message when no PS/2 Mouse and add-in card using IRQ 12.

o	Adjusted Resource Auto-configuration order for Windows 95 so that Motherboard Resources will be allocated before PCI resources.  This has been implemented via a SETUP option.

o	Adjusted PCI Resource allocation related to Video IRQ assignment.  We can now, on a product by product basis, decide to not assign IRQ’s to PCI Video devices.

o	Fixed the programming of the Cache Line Size and Latency Timer for Bridges.

o	Fixed Motherboard Device Node Attributes to indicate “device can only be configured at runtime” from “device can be configured at runtime”.  This fixes issue with WINICU R1.41 which thought the motherboard devices could be reconfigured for a next boot situation but the System Bios doesn’t support this.

o	Fixed SETUP code to make sure one interrupt was preserved for PCI usage.

Beta 2.01

o	Fixed 32 bit PCI Set Irq to check for valid PCI Busses.  PCI Local Bus Specification Revision 2.1 change.

o	Fixed PCI and ISA PnP auto-configuration code so that a Configuration Error is flagged and the device is not disabled (DSC #3394).

o	Adjusted ISA PnP auto-configuration code so that back-to-back Isolation is not performed.  This helps certain ISA PnP cards which cannot handle this.

o	Fixed jump to wrong label in ISA PnP auto-configuration code.

o	Fixed PCIDIAG V1.2 hang and extraneous PCI.EXE entries.

Release 1.0

o     None.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     Updated Device Node information for onboard IDE devices to describe them as Bus Master devices whhcih are compatible with standard IDE devices.

Beta 1.01

o     Added support to allow Chicago to init all devices except Boot devices.

o     Fixed Device Node allocation error messages.

o     Turned of Multi-function Header bit in PIIX PCI Configuration Space Header byte since PIIX IDE Function #1 is defined as a Device Node.

o     Device Node added for PIIX IDE Controller.

o     Device Node added for ports 78h, 79h, & Bus Master IDE (0FFF0h-0FFFFh).

o	See APM below.

o	Plug and Play version 1.2 will be supported in the Phase III BIOS.

APM 1.1

Release 3.0

o	None.

Release 2.0

o	Added enhancement to APM code to allow audio feedback (beeping) which indicates APM code detection of:

a)	Sleep/Resume/Soft Power Off button press (single beep),

b)	Soft Power Off via 2 second hold of Sleep/Resume/Soft Power Off button (two decending beeps).

o	Fixed problem revolving around Bios Data Area offset 6Ch (dword - 32 bit value) aka Timer Count value.  Symptom of problem fixed:  “incorrect updating of Timer Count value in Bios Data Area occurs during Sleep/Power-Down when running Windows 95”.  Fixed by utilizing Real Time Clock AND IRQ 0 (was previously only IRQ 0) as conditions for updating Timer Count value in Bios Data Area.

Beta 2.02

o	Adjusted code to always dispatch APM code reguardless of Enable or Disabled via SETUP option.

o	Fixed OS/2 trap problem related to APM being Enabled/Disabled in SETUP.

o	Fixed APM code issues related to missing interrupts.

o	Fixed APM code for alarm events.

o	Adjusted APM code so front panel switch being held for 3 seconds will be interpreted as a soft power off.

Beta 2.01

o     None.

Release 1.0

o     Removed Soft Power Off setup option now supported in hardware.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     Decreased time for system to power down by approximately 2 seconds, once front panel switch is pressed..

Beta 1.01

o     Updated APM fast off timer programming.

o     Updated code for Front Panel sleep switch.

o     APM code no longer flashes keyboard LEDs.

o     Added support to handle Soft Power Off.

o     Updated IDE Hard Disk Table Support code to allow APM to function properly with new ATA IDE code.

o     See Setup above.

o	Fixed Chicago Front panel sleep operation.

o	Advanced Power Management that supports both the APM 1.0 and 1.1 specifications.

o	Standby and Suspend are handled as like events allowing power down from OS/2 Control Panel.

o	Critical 1.1 calls are allowed even if the OS connects as 1.0.  This allows POWER.EXE to run in 1.0 while a custom Windows driver takes advantage of full 1.1 compatibility.

o	Setup menus are still TBD.

New Int 15h Extensions

Release 3.0

o	None.

Release 2.0

o	None.

Beta 2.02

o	None.

Beta 2.01

o     None.

Release 1.0

o     None.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     None.

Beta 1.01

o	New Int 15h Extensions for Extended Memory and Memory Map have been added.

o	Function E801 has been added to report the amount of contiguous memory above 1Mb.

o	Function E820 has been added to report a full memory map of the system.  This function is used by Chicago and newer versions of Windows NT.

Other Changes

Release 3.0

o	Fixed problem where video bios was disabling palette snooping. Video bios revision 1.03.08.12-57Mhz.

Release 2.0

o 	Fixed problems with S3 Trio 32 Video failures via reduction in Video Bios Clock rate reduction from 60Mhz to 57Mhz.  Also corrected font issues.  Video bios revision 1.03.08.09-57Mhz.

o	Fixed problem with ISA Shared Memory Setup options which was introduced between Beta 2.01 and Beta 2.02.  Now back to Beta 2.01 and Release 1.0 operation.  Failure was characterized by a failure of PCI video (with video error beep) when one would set the ISA Shared Memory option to start at C8000h.

Beta 2.02

o	Fixed Bios Error Display code to Not Wait for User Input if no Keyboard was detected.

o	Fixed Bios Error Display code to Not Wait for User Input if no Video was detected (DCS #3401).

o	Fixed issue related to stack handling during compatible Shutdown 9.

o    	Added new Crystal Audio micro-code.  Fixes problems of hangs at boot time and issues with Magic Carpet Software.

Beta 2.01

o	Added Product Information Storage and Retrieval features

o    	Added new Crystal Audio micro-code.

Release 1.0

o     None.

Beta 1.04

o     None.

Beta 1.03

o     None.

Beta 1.02

o     None.

Beta 1.01

o	None

Known Errata With This Release

o	IDE CD-ROM only works as IDE Slave or on a channel by itself.  It will not function as a master.

Features Still To Be Added / Comments About This Release

o	CD-ROM Boot Support

o	ATAPI Support

Notes:

Note 1:
The various O/Ss either do their own memory detection or issue a series of BIOS interrupts to determine memory size. The most common series of BIOS interrupts are INT 12 to get base memory.  Int 15 AH=88h is issued to get extended memory. Int 15h AX=E801h is issued to get memory above 64MB. NT-Daytona Beta 2 and Chicago issue Int 15h AX=E820. 
A problem occurs because there is no standard as to what Int 15h AH=88 returns. Microsoft expects up to 63MB UNLESS E801 is supported , when they expect only 15MB maximum. If Microsoft sees greater than 15MB they limit the memory to the AH=88 value and ignore the E801 results. Some O/Ss in the above situation limit memory to 16MB. Some O/Ss excpect Int 15 AH=88 to return up to 63MB and look at E801 for memory above 64MB.

Eliminating the Int 15h AX-E801 support allows all O/S to handle up to 64MB using the Int 15h AH=88h. Those O/S that support Int 15h AX=E820h will handle memory above 64MB