This is version 1.14 of the RVGNDIS driver, dated 6-Jun-96.     It is for

use by 100VG/AnyLAN Ethernet cards running NDIS Version 2 systems.

Beginning with version 1.14, "shared-memory" support has been deleted for

all cards  (it was confusing and did NOT give a performance improvement).

Version 1.14 of the driver will therefore ignore  any SHAREDMEMORY or MEM

keywords in the user's PROTOCOL.INI file and will initialize all cards to

run in "I-O mode".

PCI cards may share an IRQ level with other devices, if required.    Note

that this should be done only on "loaded" systems where there is no other

way of configuring the system,  since sharing an IRQ level among multiple

devices is inefficient and costs some performance.   Note also that older

OS/2 versions will REFUSE to allow a shared interrupt for PCI cards (such

versions assume only an EISA or MicroChannel card may share interrupts!).

In this case, the driver will attempt to set up a PCI card's interrupt as

"unshared" and,  if this is successful,  the driver displays a message to

that effect and runs the PCI card anyway.     On such older OS/2 systems,

the user must ensure that each 100VG PCI card is actually assigned to its

own IRQ level, or the system will crash!!

Microsoft LAN Manager V2.x Installation --

Follow the installation procedures in the LAN Manager Installation

Guide.   The "Importing Additional Network Device Drivers" section

describes the process of loading a driver.    The "Network Drivers

Dialog Box" section describes how to specify which driver to use.

Microsoft LAN Manager V1.0 Installation --

Install LAN Manager  as described in the LAN Manager  installation

guide, if you have not already done so.     Then, do the following

steps to install the 100VG-AnyLAN adapter drivers:

1.  Copy the RVGNDIS.OS2 driver and RVGNDIS.MSG message file  from

the installation diskette to the desired hard-disk directory.

2.  Add the following line to the CONFIG.SYS file:

DEVICE=[drive:directory]RVGNDIS.DOS

This line should be entered  immediately after the line which

installs the PROTMAN.OS2 device driver.

3.  Add RVGNDIS_NIF to the bindings list  of the protocol in the

PROTOCOL.INI file.   An example is as follows:

[NETBEUI_XIF]

DriverName = NETBEUI$

Bindings = RVGNDIS_NIF

4.  Add the following lines at the end of the PROTOCOL.INI file:

[RVGNDIS_NIF]

DriverName = R100VG$

IOADDRESS = 0xNNNN

.  .  .

.  .  .

(Other parameters as required; see below)

.  .  .

.  .  .

NOTE:  The IOADDRESS parameter in the PROTOCOL.INI file  is REQUIRED

for most CPUs; if unspecified, a default ISA address of 0x300

will be used!

Driver Parameters --

The PROTOCOL.INI file defines the configuration of protocol  and

NDIS drivers.     The following table summarizes all the RVGNDIS

parameters.    For additional information about the PROTOCOL.INI

file  and network device drivers,  see the Microsoft LAN Manager

Network Device Driver Guide.

�����������������������������������������������������Ŀ

�                �                       �            �

�  Parameter     �         Range         �  Default   �

�                �                       �            �

�����������������������������������������������������Ĵ

�                �                       �            �

�  DRIVERNAME    �  R100VG$ or R100VG$n  �  Required  �

�                �                       �            �

�  IOADDRESS     �  0x100 - 0xFFE0       �  Required  �

�                �                       �            �

�  INTERRUPT     �  3 - 15               �   EEPROM   �

�                �                       �   or PCI   �

�                �                       �            �

�  NETADDRESS    �  "000000000000" thru  �   EEPROM   �

�                �  "FEFFFFFFFFFF"       �            �

�                �                       �            �

�������������������������������������������������������

DRIVERNAME

Identifies the network device driver.  The DRIVERNAME is R100VG$

for the first driver, R100VG$2 for the second, etc.    Note that

a separate driver is required in NDIS version 2 for each adapter

card.   This parameter is required.

IOADDRESS

Identifies the I-O address of the adapter.    For EISA adapters,

the I-O address is 0x?C38, where ? is the EISA slot in which the

adapter is located (0x1C38 for slot 1, 0x2C38 for slot 2, etc.).

For ISA adapters, the I-O address is programmed into the adapter

by the VGCFG.EXE utility;  and for PCI adapters, the I-O address

must be determined using the FINDPCI.COM utility  to examine the

system's PCI BIOS.    If not specified, a default ISA address of

0x300 will be used.    This parameter is required.

INTERRUPT

This entry overrides  the EEPROM-programmed interrupt level  for

ISA and EISA adapters.     It must be a decimal number between 3

and 15.   Note that interrupt level 8 is illegal, and some other

levels may be required for other peripherals.     This parameter

is always ignored by PCI adapters,  which must use the interrupt

level assigned to them by the PCI BIOS.    This parameter should

be used by ISA and EISA adapters only when an override is needed

(e.g., for diagnostic testing).

NETADDRESS

This entry permits the user to override the adapter node address

programmed in the 100VG EEPROM.     The entry must be a 12-digit

string of hexadecimal characters and must be enclosed in quotes.

Also, the low-order bit of byte 0 must be zero.

Revision History --

Version 1.10:  Added 7001 PCI card support.

Version 1.11:  Fixed 100VG config-register usage during resets.

Version 1.12:  Added an adapter re-init in case of input errors.

Version 1.13:  Fixed certain OS/2 initialization errors.