SYNC570.LAN v 2.0.5 BETA

Installation instructions for the SYNC570.LAN driver.

The installation of the WHSM driver for the Arnet SYNC/570

and SYNC/570i boards is very simple.

This release contains two files --

SYNC570.LAN

SYNC570.LDI

Copy these files to the SYSTEM directory of your Novell Server.

At this point you may want to install the driver into MPR (3.0 or 3.1).

To do this load INETCFG.NLM on the system console.

When INETCFG.NLM is loaded select the "Boards" option.

This will bring up a list of lan and wan adapters currently installed

on your system.

Press the "Insert" key to bring up a list of available drivers.

Part way down the screen should be a driver titled "SYNC570".  Select

that.

The next screen is the Board Configuration Screen.

You must enter a Board Name.

Select the PCI board number from the list --

0, 1, 2, 3, 4, 5, 6, 7

0 means you are actually using an ISA card in which case the remaining

parameters must be specified.  A value of 1 - 7 signifies PCI adapter 1 - 7

in which case the remaining parameters are ignored;  the actual values

are read from PCI configuration space.

Then select your desired interupt level from the following list --

3, 5, 7, A, B, C, F

Select an IO Port address (whichever one you selected when you

installed the adapter card in the system) from the following list --

100, 110, 120, 130, 140, 150, 160, 170

180, 190, 1a0, 1b0, 1c0, 1d0, 1e0, 1f0

200, 210, 220, 230, 240, 250, 260, 270

280, 290, 2a0, 2b0, 2c0, 2d0, 2e0, 2f0

300, 310, 320, 330, 340, 350, 360, 370

380, 390, 3a0, 3b0, 3c0, 3d0, 3e0, 3f0

Select an unused Memory address for the host adapter --

A0000, A4000, A8000, AC000, B0000, B4000

B8000, BC000, C0000, C4000, C8000, CC000

D0000, D4000, D8000, DC000, E0000, E4000

E8000, EC000, F0000, F4000, F8000, FC000

C00000, C40000, C80000, CC0000

D00000, D40000, D80000, DC0000

E00000, E40000, E80000, EC0000

F00000, F40000, F80000, FC0000

The comment field is irrelevant.

For a Micro Channel board the only option to select is the board

Slot.  Select the slot number where the Micro Channel version of

the SYNC/570 is located.   All other parameters are determined by

the POS utility.

Press "Escape" and select "Yes" and the board will be installed.

See the Novell MPR 3.0 manual for details on configuration of the

Network Interface, Protocols, Bindings and other options.

One important item to note is that the configuration assumes a 4

port board.  If a 2 port board has been installed the user should

simply not enable the last two ports.

The load line for the board will be added by INETCFG automatically.

The syntax for the line is ....

load SYNC570.LAN port=xxx mem=yyyyy int=z channel=a name=syn_1

or, for a Micro Channel machine, its ....

load SYNC570.LAN slot=1 channel=a name=sync570_1

When the system is rebooted if the driver does not initialize correctly

one of the following errors may be generated.

SYNC570: Fatal Error: Duplicate Memory Address.

Another adapter has already registered the configured

SYNC/570 memory address.  Please select another address.

SYNC570: Fatal Error: Maximum Number of Boards exceeded.

There driver is capable of supporting a maximum of four

SYNC/570 adapters.  That limit has been exceeded.

SYNC570: Fatal Error: Config File Not Found, Exiting

The Name= parameter of the board load line has not

been configured.  Check to see that the driver load line

(ex. "load SYNC570.LAN port=120 mem=d8000 int=5 channel=1 name=fred")

is the same as the configured board.

SYNC570: Fatal Error: Incorrect Major or Minor Version Numbers.

The Major and Minor version numbers of the TSM are

invalid.  Check your MPR 3.0 installation.

SYNC570: Fatal Error: TSM Port Registered Error.

Registration with the TSM failed.  Check to see

that the TSM has loaded.  Check the board configuration.

SYNC570: Fatal Error: Bad Channel/Line No Param.

An invalid channel number has been selected.  An example

would be trying to install channel 3 of a 2 port board.

Check your board installation.

SYNC570: Fatal Error: Channel already in use.

The selected channel has already been configured.

SYNC570: Fatal Error: SNMP Registration Failed.

The SNMP support was unable to register.

SYNC570: Fatal Error: No Port Addr Param.

The port address was not included on the command line.

Make sure a port=xxx parameter is included on the load

line.

SYNC570: Fatal Error: Unable to register with WSM.

The WAN board (the SYNC/570 driver specifically) was unable

to register with the upper levels of the protocol stack.

Check the board configuration.

SYNC570: Fatal Error: Unable to Initialize Board.

Board initialization failed.  Run diags on the board.

SYNC570: Fatal Error: Unable to Initialize Port.

Port Initialization failed.  Check for a bad board.

Check port configuration in INETCFG.

SYNC570: Fatal Error: Unable to get TSM entries.

The driver was unable to get the TSM entry points.  Check

board and port configuration along with MPR installation.

SYNC570: Fatal Error: Unable to get resource tag.

The driver attempted to allocate a resource tag and failed.

SYNC570: Fatal Error: Unable to get memory.

The driver was unable to allocate memory for internal usage.

SYNC570: Fatal Error: Parse Driver Parameter.

Invalid parameters were selected on the load line.  Please

check the load line for accuracy.

SYNC570: Fatal Error: A null pointer was passed as an initialization parameter

The board was unable to be initialized.

SYNC570: Fatal Error: The initialization parameter is out of range

An attempt was made to initialize the board with a parameter

that was invalid.

SYNC570: Fatal Error: The Board is already initialized

The driver has been called twice for the same board and

port combination.

SYNC570: Fatal Error: The initialization parameter is in error

An attempt was made to initialize the board with an

incorrect parameter.

SYNC570: Fatal Error: Memory for the board could not be allocated

The driver was unable to allocated memory for internal

functions.

SYNC570: Fatal Error: Board RAM failed the memory test.

A memory test has been executed on the SYNC/570 and

this test has failed.  Please check that there are

no other adapters trying to use the selected memory

address.  This may indicated board failure.

SYNC570: Fatal Error: The board cannot be found at IO port.

The board does not appear to be at that IO port.  Please

check your dipswitch settings.

SYNC570: Fatal Error: The board memory address conflicts with system memory.

The SYNC/570 has been configured to be at an already

allocated memory address.  Please select another address.

There are a handful of warnings that may occur.  Please check your

configuration if these occur.

SYNC570: WARNING: Config Record Error, Using Defaults.

SYNC570: WARNING: Config File Not Found, Using Defaults.

If either of these errors occur then check the configuration

of the driver and the load line that is installing it.

SYNC570: WARNING: Maximum Packet Size smaller than Configured MRU Size using smaller value.

The Maximum internal packet size is smaller that the size configured.

The lesser of the two values is being used.