KTI 100VG LAN Adapter's ODI Driver KVGODI.SYS

for NetWare 3.12 and 4.X OS/2 Client Installation

=================================================

The OS/2 ODI driver files have been properly

prepared in the driver diskette under \OS2 sub-directory for Netware

3.12/4.X OS/2 Client installation.

Procedures:

----------

1. Start up your OS/2 system.

2. Insert the Netware Client for OS/2 Disk1 WSOS2_1 into drive A:.

3. Choose the "Drive A" icon from the desktop.

4. Choose the "Drive A" icon from the "Drive A Tree-View".

5. Select the "INSTALL.EXE" icon.

6. Select the "Installation" menu bar.

7. Select the "Requester on Workstation" bar.

8. Use C:\NETWARE as default for Requester files and select "Ok".

9. Select "Edit CONFIG.SYS and Copy All files" item.

10. Insert Adapter driver diskette inot drive A:.

11. Select "Continue".

12. "KVGODI.SYS" is listed on the screen.

13. Select "Continue".

14. Follow the screens to complete "installation" menu.

15. Select "Configuration" menu bar from the Netware Workstation for

OS/2 Installation Utility.

16. Select "This Workstation".

17. Enter the default location for NET.CFG file.

C:\NETWARE

18. Type the following lines for the NET.CFG:

Link Driver KVGODI

Frame Ethernet_802.2

19. After complete the creation of NET.CFG file, select "Save".

20. Exit "Install".

21. Shut down your system and reboot it.