HOW TO USE LCS-8834P ON LANtastic

You have to copy the LCS-8834P LANtastic driver "LSPLAN.EXE"

to the LANtastic Operating System directory. And run it

as the followings:

C:\>LSPLAN VERBOSE		/* LCS-8834P LANtastic driver */

------- ShineNet LCS-8834P LANtastic Driver  Ver 2.00  08-08-93 -------

Command line		  VERBOSE

IEEE 802.3 node address  040000001466	Network packet size	 1500

MPX interface number	 C7		IO base address 	 0378

Interrupt request (IRQ)  5		Packet type		 IEEE 802.3

Bytes of memory used	 6864		Data Mode		 0

----  ShineNet LCS-8834P LANtastic Driver installed ----

Now, You can run AILANBIO.EXE and another LANtastic untility.

Note:

LSPLAN.EXE just support Artisoft LANtastic version 4.xx

LSPLAN.EXE has been tested over AI-LANBIOS version 3.01

LANtastic is a trademark of ARTISOFT, Inc.