MS Windows for Workgroups Installation With PE-EPP Cards

========================================================

This driver diskette includes an OEMSETUP.INF file for Windows for Workgroups.

Using the OEMSETUP.INF file, the user can select whichever PE-EPP network

adapters are to be installed based on Windows dialog menus.

The OEMSETUP.INF file contains selections for all the PE-EPP series

network adapters:

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, it will show a

list of all network adapters supported by Windows.  You must select

'Unlisted or Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the PE-EPP OEMSETUP.INF

file is included in the PE-EPP driver diskette, insert the driver diskette

in drive A:

Input the full path name where the PE-EPP OEMSETUP.INF file is located.

(i.e. A:\NDIS\OEMSETUP.INF)

4. You will see a Windows dialog menu listing all PE-EPP network adapters

included in the OEMSETUP.INF file.  Select the network adapter that

you will be using.

5. After selecting the type of PE-EPP network adapter, Windows will look

for the NDIS driver of the network adapter you have selected.

At this time, the PE-EPP driver diskette should still be in drive A:.

Input the full path name where the PE-EPP NDIS driver is located.

(i.e. A:\NDIS\DOS\)

6. Windows will copy the NDIS driver file and update the Windows network

system automatically.  Be sure to configure the network adapter if you

are using I/O port or IRQ other than the default.

At this point, you can finish the installion or continue the process

according to your preferences.

7. After installation is completely finished, restart your system for the

updates to take effect.

PE-EPP NDIS PROTOCOL.INI User Guide

===================================

This guide lists the available PROTOCOL.INI keywords for all

PE-EPP NDIS device drivers.  Most of the keywords are optional

and have default values.  The asterisk symbol (*) indicates the

list of PE-EPP adapters where the keyword is applicable.

IOADDRESS - This keyword specifies the base I/O port address used

by the PE-EPP Network Interface Card. This value must

match the base I/O address of printer port on your PC.

INTERRUPT - This keyword specifies the interrupt line used by the

PE-EPP NIC. This value must match the IRQ of printer

port on PC.

DRIVERNAME- This keyword specifies the NIC driver's name.  The NIC

driver name follows the NIC adapter type.

PRINTPORT - This keyword specifies the printer port used by your

PE-EPP Pocket LAN Adapter.

NETADDRESS- This keyword specifies the node address (hex) used by

your network interface card.  This option is used only

if you want to override the node address burned in your

NIC.  In this case, the specified address will be used

instead of the NIC's actual node address.