NetWare Driver Installation 	   September 1992

===========================

The NetWare driver programs in this disk support the following PE-EPP cards:

PE-EPP,PE-EPPCT,PE-EPPTP,PE-EPPAUI,

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory	      Contents

---------	      --------------------------------------------------------

\NETWARE	      A pre-generated IPXEPP.COM

\NETWARE\NET386       NetWare 386 server or bridge drivers

\NETWARE\V215B	      NetWare v2.15b workstation drivers

\NETWARE\V215C	      NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\ODI.OS2      ODI workstation driver for OS/2

*********************************

*  QUICK SETUP FOR WORKSTATION	*

*********************************

A pre-generated workstation driver IPXEPP.COM is included in this disk.

You can simply run the IPXEPP on a workstation to access the Novell network:

>IPXEPP	--Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx	  --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXEPP.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\	--For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

EPPODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

************************************

* Install NetWare V2.2 workstation *

************************************

Refer to procedure  A. HOW TO SETUP IPX and follow all procedures

except for #3.	Instead of running the program SHGEN.EXE, run WSGEN.EXE .

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

PE-EPP ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. PE-EPP ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.	For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the EPPODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the EPPODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL		--Link Support Layer Module provided by Novell

>EPPODI		--Hardware Specific Module provided by PE-EPP

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy EPPODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\EPPODI.SYS

3)  PE-EPP ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example :  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\	--SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\386EPP.LAN     --Select your network card settings

:Bind ipx to 386EPP

:load install		--Follow the NetWare installation instructions