S.T.B. Systems Compatibility Report

1651 North Glenville

Richardson ,TX  75085             Product Tested: Sprint VL Trident

MONITOR MAKE & SIZE:NEC XV15	DATE OF REPORT: 7/21/95

C.P.U. MAKE & SPEED:Intel 33mHz	 VIDEO  BIOS  REV: Sprint VL 1.0

SYSTEM  &  BIOS   VER:UMC AMI	WIN & UTIL DRV. REV:	Win 95 Upgrade

TECHNICIANS NAME: Gene Pippenger

-----SPECIFIC HARDWARE OR APPLICATION ERRORS -----

Refresh rates established by Win 95 for the NEC XV15:

640 x 480: 60 Hz, 800 x 600: 72 Hz, 1024 x 768: 60Hz NI,

1280 x 1024 or 1600 x 1200: not offered.

Applications/Accessories Run without incident: Those that come with Win 95:

Videos, Paint, Diagnostics, DOS prompt, games, all on the same screen with no lockup.

Results of driver tests:

640 x 480 x 16 and 800 x 600 x 16 ran, oddly enough, in Monochrome, while 124 x 768 x 16 NI was not even offered. All three resolutions ran flicker free at 256 colors but no other color depths were offered.