VideoLogic GrafixStar 700 for Windows 95

29th April 1997.

Contents

1. Introduction

2. The GrafixStar 700 drivers

3. Installing GrafixStar 700

4. Removing SmartTools

5. Troubleshooting

6. Field notes

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 700 drivers.

If you are reading this file outside of the VideoLogic Installer and want to install GrafixStar 700 manually, refer to section 3 below or the User Guide for complete instructions on how to do this.

2. The GrafixStar 700 drivers

Introduction

This section describes the display modes available with video acceleration, and how GrafixStar 700 interacts with DirectDraw.

Display modes with video acceleration

The modes below support video acceleration:

Display size	Colors	Refresh rate (Hz)

640 x 480	256	60, 72, 75, 85, 90, 100, 120

800 x 600	256	56, 60, 72, 75, 85, 90, 100, 120

1024 x	768	256	43, 60, 70, 72, 75, 80, 85, 100

1152x864	256	60, 70, 75, 90

640 x 480	65,536	60, 72, 75, 85, 90, 100, 120

800 x 600	65,536	56, 60, 72, 75, 85, 90, 100, 120

1024 x	768	65,536	43, 60, 70, 72, 75, 80, 85, 100

640 x 480	16.7m	60, 72, 75, 85, 90, 100, 120

All other modes display AVI and MPEG files using software only, which gives poorer quality playback.

Using games with DirectDraw

GrafixStar 700 display drivers for Windows 95 are compatible with DirectDraw 2.

SmartTools automatically installs DirectVideo with DirectDraw. To manually install DirectVideo with DirectDraw, run GS700\WIN95\DISK2\REDIST.IHV\SETUP.EXE from the VideoLogic Software Library CD, after which Windows 95 needs to be restarted. To remove DirectDraw from your PC, delete the following files, where [drive] is the hard drive on which you are running Windows 95:

File	Location

Ddhelp.exe	[drive]:\windows\system

Dsound.vxd	[drive]:\windows\system

Dsound.dll	[drive]:\windows\system

Ddraw.dll	[drive]:\windows\system

Ddraw16.dll	[drive]:\windows\system

D3drm.dll	[drive]:\windows\system

D3dxof.dll	[drive]:\windows\system

D3dim.dll	[drive]:\windows\system

D3drampf.dll	[drive]:\windows\system

D3drgbf.dll	[drive]:\windows\system

D3drm8f.dll	[drive]:\windows\system

D3drm16f.dll	[drive]:\windows\system

D3drm24f.dll	[drive]:\windows\system

D3drm32f.dll	[drive]:\windows\system

D3drg8f.dll	[drive]:\windows\system

D3drg16f.dll	[drive]:\windows\system

D3drg24f.dll	[drive]:\windows\system

D3drg32f.dll	[drive]:\windows\system

D3dhalf.dll	[drive]:\windows\system

Dplay.dll	[drive]:\windows\system

Dpserial.dll	[drive]:\windows\system

Dpwsock.dll	[drive]:\windows\system

Dvideo.dll	[drive]:\windows\system

3. Installing GrafixStar 700

Introduction

The easiest way to install your GrafixStar 700 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the CD into your PC. However, if you don’t have a User Guide, or you want to install the software manually, you should refer to the following instructions.

Handling GrafixStar 700

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 700 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 700 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 700. All cables connected to GrafixStar 700 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation will vary slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the System icon to open the System Properties Window.

3.	Ensure that the General tab is selected. This page has details about the System, Registration information, and Computer details. The build number is located in the System section, under the ‘Microsoft Windows 95’ line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs700\win95\disk1 where [disk] is your CD drive.

8.	Click GrafixStar 700 GrafixStar from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

12.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

13.	Follow the instructions that appear on the screen.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6.	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7.	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs700\win95\disk1 where [disk] is your CD drive.

10.	Click GrafixStar 700 GrafixStar from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

14.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

15.	Follow the instructions that appear on the screen.

4. Removing SmartTools

Note: Save any open documents before removing SmartTools, as you will need to restart Windows.

To remove SmartTools:

1.	Click the Start button, and point to Settings.

2.	Click Control Panel.

3.	Click Add/Remove Programs.

4.	From the list of programs displayed, click VideoLogic SmartTools, and click Add/Remove.

5.	Follow the instructions that appear on the screen.

5. Troubleshooting

Introduction

This section describes problems you may encounter when using SmartTools or running GrafixStar 700.

Display Properties

Errors when running virtual desktops and/or changing modes

Some applications, especially those that display video, report errors when running a virtual desktop or changing modes. Every effort is made to prevent such problems but if they do occur we recommend that you change to a non-virtual desktop mode, or close the problematic application before changing modes.

Modes

16-color modes

SmartTools is not designed to work with a 16 color palette (4 bits per pixel). You must change to a higher palette via the Display Settings dialog box of Control Panel to enable SmartTools to work.

Monitors

Display sizes

Windows 95 does not restrict the generic SVGA monitor settings to a given set of display sizes, so SmartTools allows any display size to be selected. If you have a standard monitor selected, there is therefore a possibility that you could zoom out to a display size that your monitor does not support.

Toolbars

Disappearing toolbars

When using SmartDesktop to zoom into the desktop, you may lose sight of the SmartTools toolbars. You can relocate a toolbar within your current display using the Find Now feature. This is located by clicking the Properties button of the toolbar on the Toolbar Properties page of SmartTools. Click the Find Now button to move the active SmartTools toolbar to a visible location within the display.

Toolbar Properties page of SmartTools

Maximize in Display

The SmartDesktop Maximize on Display utility maximizes your applications to the full size of your display, so part of your application window may disappear behind the Windows 95 taskbar. If you use this feature, we recommend that you set the taskbar to auto-hide.

Moving and deleting buttons from toolbars

You can move buttons around on the toolbars by holding down the SHIFT key and dragging them to the desired location. To remove a button, drag it off the toolbar while holding down the SHIFT key.

HotKeys

DOSBox

The SmartDesktop hotkeys will not function when a DOSBox is active.

Connectivity - Feature connector

Feature connector

If you select Feature Connector from the Connectivity page of SmartDisplay, your CONFIG.SYS file is modified to include the TSR VLENABLE.SYS. VLENABLE is a utility that enables DAC snooping when you turn on your PC.

If your system does not have a CONFIG.SYS file, or it has one that is zero bytes in size, you must create a file called CONFIG.SYS at the root of your hard disk and make sure that it contains some data (a rem line is sufficient), before you select Feature Connector.

When you have selected Feature Connector you must restart your system to run VLENABLE.

If you uncheck the Feature Connector option, the VLENABLE.SYS line in the CONFIG.SYS file will be disabled by changing it to a rem line.

Drivers

QuickTime for Windows Version 2.X

If you experience problems when viewing QuickTime movies, run the QuickTime Control Panel from the Windows 95 Control Panel, and follow the steps below:

1.	Run the QuickTime Control Panel, click the More button and click the Video tab.

2.	From within the Optimization section, the Draw Method setting will be set to Video Hardware. Change this to Video Driver.

3.	Close the QuickTime Control Panel.

Playing AVI with MS PLUS! pack

If you are using a display mode which is capable of video acceleration and you have the Show Window Contents While Dragging option enabled, you will see the accelerated video displaced from the video window while dragging the video window around the desktop. Disable the Show Window Contents While Dragging on the PLUS page in Display Properties.

Monitor screen centering

Some advanced monitors create a display setting by detecting the horizontal line frequency, and this may change the size and positioning of the Windows 95 screen when task switching from a DOS full-screen session to a Windows 95 session. This problem occurs when the monitor is at the borderline between two settings. To remedy this, use the controls on your monitor to adjust its screen settings.

Quickres

Quickres is part of a series of unsupported utilities provided by Microsoft. The mechanism used by Quickres to achieve color-depth switching on the fly can reduce GrafixStar 700 performance, and make Windows 95 unstable. We recommend that you do not use this utility.

Xing 3.X

The latest version of Xing MPEG player is v3.12. We recommend that you use this version instead of v3.0, as v3.12 includes a few bug fixes. There is an update from Xing v3.0 to v3.12 provided on the VideoLogic Software Library CD v4 or later. You can install the update by running [drive]:\xing\xmp312p.exe of the VideoLogic Software Library CD-ROM.

MPEG clips replayed in modes other than the video accelerated modes may cause Windows to become unstable. If this problem occurs, change the display mode to one that supports video acceleration.

6. Field notes

Introduction

This section gives a summary of VideoLogic field notes. Field notes contain frequently asked questions, troubleshooting information, late-breaking information or general comments about GrafixStar 300. Where possible software patches have been provided.

Field notes file format

All of the field notes are in Adobe Acrobat format, and have the extension .PDF. To read these files you must first install the Adobe Acrobat reader which is supplied on the CD. It is located in the directory [drive]:\reader\win95 where [drive] is your CD drive. When you have installed the reader, double-click the .PDF file to read the field note, or open it from within the reader.

All of the patch descriptions are in DOS .TXT format.

Field notes folder

All the notes are located in their own folder within the \UPDATES folder on the CD, so that, for example, field note 156 is in the folder GS700\UPDATES\FN0156. Patches are in their own folders within the field note folders. For example, patch 92 is in the folder GS700\UPDATES\FN0156\PATCH92.

Field Note	Summary	Patch

FN0143	Using GrafixStar cards on Compaq XL PCs

FN0156	VLMEMCHK.SYS (Updated to include fix for NEXGEN PCs)	92

FN0192	Monitor Selection Problems	106

FN0193	Color corruption or blank screen with IBM 350 P9	107

FN0199	DESK.CPL error with SmartTools

FN0204	Award BIOS (Intel EtherExpress PRO/100 LAN Adapter)

FN0212	Windows 95 Version 4.1.0 Drivers for VLB graphics cards	212

FN0245	Loss of sync on Compaq PCs	245